Precipitation observation data set of Jiuyuangou watershed in the first sub region of Loess Hilly and gully region from 2015 to 2020

DOI：10.12072/ncdc.HWSDZhan.db0038.2020

CSTR：11738.11.ncdc.HWSDZhan.2020.96

Share type:Apply to Access

Category:Soil and Water Conservation

Datasets description

This data set is the observation data of soil moisture content in runoff field of different measures in yulinggou watershed, the first sub area of Loess Hilly and gully region from 2015 to 2020. For the basic information of runoff field, see the description of runoff field with different measures in yulinggou from 2015 to 2020

Data source description

<p>Instrument continuous observation Manual timing observation

Data processing method

<p>Precipitation observation instruments include SM1 ordinary rain gauge with a diameter of 20cm and dsj2 siphon type self recording rain gauge. The SM1 type adopts the 8-hour and 20-hour two-stage system for annual observation. It mainly observes the start and end time of precipitation and makes statistics of precipitation. When the intermittent time of precipitation is more than 15 minutes, it is observed as two times of precipitation before and after the interval. When the intermittent time is less than 15 minutes, it is recorded as one-time precipitation observation. If one-time precipitation crosses 8 hours and 20 hours, it should be measured at 8 hours and 20 hours. Dsj2 type adopts 24 section system for flood season observation. In case of failure of self recording rain gauge, the observation data of ordinary rain gauge is used for supplement. </p>

Data quality description

<p>In the process of observation, when there is an abnormal value of the self recording rain gauge, the observation value of the ordinary rain gauge is used for interpolation and verification; in the process of data entry, there are four levels of manual verification, which are tabulation, primary calibration, recalibration and audit, to ensure the data quality. </p>

Base information

collect time:2015/01/01-2020/12/31

collect place:Xindiangou watershed, Suide County, Yulin City, Shaanxi Province

altitude:2.0m- Nonem

data size:97.3 KiB

data format:

Coordinate system: WGS84

Keywords

Theme:Daily precipitation, Period precipitation, Precipitation intensity
	Subject:
	Time:2015 2016 2017 2018 2019 2020
	Place:Yulin,  Xindian ditch,  Suide,  Shaanxi Province

Space scope

|  | north： 37.52 |  |
| --- | --- | --- |
| west：110.28 |  | east：110.30 |
|  | south： 37.49 |  |

Service information

contacts:Min Yu Fang

phone:093149675976

mailbox:myf@lzb.ac.cn

Contact information

contributors:Suide administration and Supervision Bureau of water and soil conservation of the Yellow River (Suide scientific experimental station of water and soil conservation)

author:LEI Xin

manager:GUO Yumei , Suide administration and Supervision Bureau of water and soil conservation of the Yellow River (Suide scientific experimental station of water and soil conservation)

Project information

Citations and annotations

In order to protect the rights and interests of the platform's scientific and technological resources, expand the services of the platform center, and enhance the application potential of scientific and technological resources,resource users are requested to mark and quote the research results (including published papers, treatises, data products, unpublished research reports, data products, etc.) generated by the use of resources in the following manner.

For the achievements published in English, please refer to the following specifications：

The dataset is provided by National Cryosphere Desert Data Center(https://www.ncdc.ac.cn).

Data reference:

Suide administration and Supervision Bureau of water and soil conservation of the Yellow River (Suide scientific experimental station of water and soil conservation). Precipitation observation data set of Jiuyuangou watershed in the first sub region of Loess Hilly and gully region from 2015 to 2020. National Cryosphere Desert Data Center(http://www.ncdc.ac.cn), 2021. https://cstr.cn/CSTR:11738.11.ncdc.HWSDZhan.2020.96.