30m slope and aspect data of the permafrost region in Northeast China (2000)

DOI：10.12072/ncdc.nieer.db7258.2026

CSTR：11738.11.NCDC.NIEER.DB7258.2026

Share type:Login to Access

Category:Frozen soil

Datasets description

Data source description

Independently processed (based on publicly available high-resolution DEM images).

Data processing method

Data extraction: Using terrain analysis modules such as ArcGIS/QGIS, the third-order inverse sum of squares weight algorithm (Horn algorithm) is used to calculate the height difference between each central pixel and its neighboring pixels.     Slope calculation: Extract the maximum ground slope angle of each pixel, accurately identify the core area of the Daxing'an Mountains main vein at 13 ° -20 °, and locally steep slopes exceeding 60 °.     Slope calculation: Calculate the azimuth angle of the horizontal projection direction, used to analyze the "yin-yang slope" effect.     Projection conversion: To reduce the area deformation in high latitude regions, it is uniformly converted to Albers equal area cone projection.

Data quality description

The basic digital elevation model (DEM) used in this dataset is derived from the Shuttle Radar Topography Mission (SRTM). The spatial resolution of the data is 1 arc second (approximately 30 meters), the elevation reference is the WGS84 ellipsoid, and the vertical reference is the EGM96 geoid.     In this study, DEM data was processed through the following process: bilinear interpolation resampling method was used to unify it into a 30m spatial resolution grid; Calculate slope and aspect parameters based on GIS terrain analysis module, with slope unit in degrees (°) and aspect represented by clockwise azimuth angles ranging from 0 ° to 360 °; All raster data is uniformly projected onto the WGS 84/UTM Zone 52N coordinate system to ensure spatial consistency.

Base information

collect time:2000/01/01-2000/12/31

collect place:Northeast Permafrost Region

data size:7.7 GiB

data format:*.tif

Coordinate system: 无

Keywords

Theme:Digital elevation model, slope, aspect
	Subject:geography 
	Time:2000
	Place:Northeast Permafrost Region

Space scope

|  | north： 53.00 |  |
| --- | --- | --- |
| west：115.00 |  | east：135.00 |
|  | south： 42.00 |  |

Service information

contacts:Min Yufang

phone:0931-4967596

mailbox:ncdc@lzb.ac.cn

Contact information

contributors:JIN Huijun,  Tang Jianjun,  Wang Wenhui,  Jin Xiaoying,  Li Shanzhen,  Huang Shuai,  Chen Dun,  WANG Hongwei,  Yangsui Bridge,  Li Zuwang,  Soaring Swallows,  Cheng Yaohui,  Li Jingtao,  Zhang Ze,  Wang Lifeng,  Zhang Hu,  Liu Mengxin,  Zhang Shengrong,  Yang Xue,  Liu Zirui,  Yue Ziying,  Wu Haibin,  Xing Luning,  Chen Siyu,  Xu Jingyan,  He Zhengyun,  Mi Hongqi,  Peng Wenhao,  Liang Junhe,  Shi Yanling,  Zhou Zhiyi

author:JIN Huijun ,  Tang Jianjun ,  Wang Wenhui

manager:JIN Huijun

Project information

number:2022FY100700，
      name:Survey of Permafrost Conditions and Freeze-Thaw Damage in the High-Latitude Regions of Northeast China

Citations and annotations

In order to protect the rights and interests of the platform's scientific and technological resources, expand the services of the platform center, and enhance the application potential of scientific and technological resources,resource users are requested to mark and quote the research results (including published papers, treatises, data products, unpublished research reports, data products, etc.) generated by the use of resources in the following manner.

For the achievements published in English, please refer to the following specifications：

The dataset is provided by National Cryosphere Desert Data Center(https://www.ncdc.ac.cn).

Data reference:

JIN Huijun, Tang Jianjun, Wang Wenhui, Jin Xiaoying, Li Shanzhen, Huang Shuai, Chen Dun, WANG Hongwei, Yangsui Bridge, Li Zuwang, Soaring Swallows, Cheng Yaohui, Li Jingtao, Zhang Ze, Wang Lifeng, Zhang Hu, Liu Mengxin, Zhang Shengrong, Yang Xue, Liu Zirui, Yue Ziying, Wu Haibin, Xing Luning, Chen Siyu, Xu Jingyan, He Zhengyun, Mi Hongqi, Peng Wenhao, Liang Junhe, Shi Yanling, Zhou Zhiyi. 30m slope and aspect data of the permafrost region in Northeast China (2000). National Cryosphere Desert Data Center(http://www.ncdc.ac.cn), 2026. https://cstr.cn/CSTR:11738.11.NCDC.NIEER.DB7258.2026.