Heihe River eco hydrological remote sensing experiment: 2m DEM data set of Dayekou basin (2012)

DOI：10.12072/ncdc.nieer.db3541.2023

CSTR：11738.11.ncdc.NIEER.2021.47

Share type:Apply to Access

Category:Remote Sensing

Datasets description

Through data programming, worldview-2 stereo pair data of Dayekou basin were obtained in mid May 2012. From July to August of the same year, the data of 27 GPS image control points and check points were measured on the basis of the basin GPS Differential geodetic control network. Based on the whole field GPS ground control points, the RPC file of worldview-2 image is corrected by using the digital photogrammetry software system. On the stereo model, 60 evenly distributed high-precision image connection points are matched through image automatic matching technology to quickly extract 1m and 2m digital elevation models (DEM) of Dayekou sub watershed in the upper reaches of Heihe River Basin. At the same time, DEM is edited in key areas such as yinpo forest covered area and Dayekou reservoir, and a certain number of feature points and line data are measured in places with large changes in topographic features, which greatly improves the accuracy of results. The quantitative DEM verification is carried out through the inspection points composed of field control points and model security points. The mean square error of elevation is 1.9m and 1.2m respectively, which meets the accuracy requirements of 2m in high mountains with a scale of 1:2000

Data source description

Through data programming, worldview-2 stereo pair data of Dayekou basin were obtained in mid May 2012. From July to August of the same year, the data of 27 GPS image control points and check points were measured on the basis of the basin GPS Differential geodetic control network

Data processing method

Based on the whole field GPS ground control points, the RPC file of worldview-2 image is corrected by using the digital photogrammetry software system. On the stereo model, 60 evenly distributed high-precision image connection points are matched through image automatic matching technology to quickly extract 1m and 2m digital elevation models (DEM) of Dayekou sub watershed in the upper reaches of Heihe River Basin. At the same time, DEM is edited in key areas such as yinpo forest covered area and Dayekou reservoir, and a certain number of feature points and line data are measured in places with large changes in topographic features, which greatly improves the accuracy of results.  The quantitative DEM verification is carried out through the inspection points composed of field control points and model security points. The mean square error of elevation is 1.9m and 1.2m respectively, which meets the accuracy requirements of 2m in high mountains with a scale of 1:2000

Data quality description

Good data quality

Base information

collect time:2012/04/14-2012/05/14

collect place:Heihe River Basin, Dayekou basin, upstream cold region, hydrological test area

data size:254.8 MiB

data format:

Coordinate system: WGS84

Keywords

Theme:Remote sensing products, DEM, coverage
	Subject:geography 
	Time:2012
	Place:Heihe River Basin,  Dayekou basin,  upstream cold region

Space scope

|  | north： 38.58 |  |
| --- | --- | --- |
| west：100.22 |  | east：100.31 |
|  | south： 38.44 |  |

Service information

contacts:Min Yufang

phone: 0931-4967596

mailbox:ncdc@lzb.ac.cn

Contact information

contributors:Ma mingguo

author:Ma mingguo

manager:Ma mingguo

Project information

number:91125004，
      name:

Citations and annotations

In order to protect the rights and interests of the platform's scientific and technological resources, expand the services of the platform center, and enhance the application potential of scientific and technological resources,resource users are requested to mark and quote the research results (including published papers, treatises, data products, unpublished research reports, data products, etc.) generated by the use of resources in the following manner.

For the achievements published in English, please refer to the following specifications：

The dataset is provided by National Cryosphere Desert Data Center(https://www.ncdc.ac.cn).

Data reference:

Ma mingguo. Heihe River eco hydrological remote sensing experiment: 2m DEM data set of Dayekou basin (2012). National Cryosphere Desert Data Center(http://www.ncdc.ac.cn), 2021. https://cstr.cn/CSTR:11738.11.ncdc.NIEER.2021.47.

Article citation:

Tao Che, Xin Li, Shaomin Liu, Hongyi Li, Ziwei Xu, Junlei Tan, Yang Zhang, Zhiguo Ren, Lin Xiao, Jie Deng, Rui Jin, Mingguo Ma, Jian Wang, Xiaofan Yang. Integrated hydrometeorological, snow and frozen-ground observations in the alpine region of the Heihe River Basin, China[J]. Earth System Science Data, 2019, 11, (3).