Dataset of Lake Area and Volume Changes in Internal Flow Basins of the Qinghai Tibet Plateau (1989-2019)

DOI：10.5281/zenodo.5543614

CSTR：11738.11.NCDC.ZENODO.DB4055.2023

Share type:Login to Access

Category:Hydrology

Datasets description

The change in lake water storage capacity is a unique indicator of natural climate change in the Qinghai Tibet Plateau (TP). However, the region still lacks comprehensive lake water storage data, especially for lakes smaller than 10 square kilometers. In this dataset, we utilized Landsat imagery and digital terrain models to conduct an annual relative lake volume (RLV) survey of 976 lakes with an area greater than 1 square kilometer in the Qinghai Tibet Plateau Internal Flow Basin (EBTP) from 1989 to 2019. Our method first identifies individual lakes, determines their analysis range, and calculates the annual lake area based on land remote sensing satellite images. Then derive the relationship between lake area and altitude, estimate the lake surface altitude, and calculate RLV. After verification and comparison with several existing datasets, it was found that our data is more reliable and comprehensive. Our research provides complete long-term lake water volume change data for the region, which is a supplement to the existing lake dataset.

Data source description

The data used in this study includes land satellite images, global surface water data from the Joint Research Center, digital elevation models (DEM) from the Space Shuttle Radar Topography Mission (SRTM), advanced land observation satellites (ALOS) digital surface models (DSM), and some public lake storage data. The spatial resolution of ALOS and SRTM is both 30 meters, covering the study area.

Data processing method

In this study, the calculation of relative lake volume (RLV) was divided into two steps. The first step is to identify individual lakes, determine their analysis range, and calculate the annual lake area based on land remote sensing satellite images. The second step is to derive the relationship between lake area and altitude, estimate the elevation of the lake surface, and calculate the changes in lake water volume. The specific steps of the first step include three sub steps: lake identification, lake analysis scope and seed determination, water area classification and segmentation, and annual lake area calculation. The second step can be divided into deriving the lake area elevation relationship and calculating the lake RLV.

Data quality description

The data quality is good.

Base information

collect time:1984/01/01-2020/12/31

collect place:Qinghai-Tibet Plateau

data size:73.8 MiB

data format:shp

Coordinate system: 无

Keywords

Theme:Qinghai Tibet Plateau, lake area, lake water volume
	Subject:geography 
	Time:1989 1990 1991 1992 1993 1994 1995 1996 1997 1998 1999 2000 2001 2002 2003 2004 2005 2006 2007 2008 2009 2010 2011 2012 2013 2014 2015 2016 2017 2018 2019
	Place:Qinghai Tibet Plateau

Space scope

|  | north： 34.43 |  |
| --- | --- | --- |
| west： 78.65 |  | east： 99.38 |
|  | south： 29.83 |  |

Service information

contacts:Min Yufang

phone:0931-4967596

mailbox:ncdc@lzb.ac.cn

Contact information

contributors:WANG Junxiao

author:WANG Junxiao

manager:WANG Junxiao

Project information

Citations and annotations

In order to protect the rights and interests of the platform's scientific and technological resources, expand the services of the platform center, and enhance the application potential of scientific and technological resources,resource users are requested to mark and quote the research results (including published papers, treatises, data products, unpublished research reports, data products, etc.) generated by the use of resources in the following manner.

For the achievements published in English, please refer to the following specifications：

The dataset is provided by National Cryosphere Desert Data Center(https://www.ncdc.ac.cn).

Data reference:

WANG Junxiao. Dataset of Lake Area and Volume Changes in Internal Flow Basins of the Qinghai Tibet Plateau (1989-2019). National Cryosphere Desert Data Center(http://www.ncdc.ac.cn), 2023. https://cstr.cn/CSTR:11738.11.NCDC.ZENODO.DB4055.2023.

Article citation:

Wang Junxiao, Wang Liuming, Li Mengyao, Zhu Liping, Li Xingong. Lake volume variation in the endorheic basin of the Tibetan Plateau from 1989 to 2019[J]. Scientific Data, 2022-10-08, 9, (1):611. 
            DOI: 10.1038/s41597-022-01711-w.