Heihe River Integrated Remote Sensing joint experiment: thermal infrared specific emissivity observation data set in ARW intensive observation area (2008)

DOI：10.12072/ncdc.NIEER.db1919.2022

CSTR：11738.11.ncdc.NIEER.2021.1911

Share type:Login to Access

Category:Remote Sensing

Datasets description

On March 14, 2008, the thermal infrared specific emissivity observation test was carried out in arrou sample 3. The measuring instrument is a portable specific emissivity meter (Patent No.: ZL 02 2 37640.2). During measurement, the quadrat is covered with 100% dry forage, and the height is less than 5cm. Since the grassland is relatively homogeneous, only the center point of each grid in arrou quadrat 3 is measured. The naming rule of each observation point is in the form of a3-9, which is expressed as the No. 9 sampling point of a rou quadrat 3. Each center point is randomly measured twice. The horizontal surface shall be selected as far as possible to ensure that the measurement angle is 45 degrees; And in strict accordance with the following requirements: （ 1) Add mirror without cover (tsky); （ 2) Mirror capping (Tcha); （ 3) No mirror, no cover (TSM) （ 4) Complete one measurement without mirror capping (TCM). To reduce the error, read 4 numbers for each observation. At the same time, the surface thermal infrared temperature (hand-held infrared thermometer) and hand-held thermal imager (FLIR thermacam) were measured simultaneously for 4 times at each central point. The calculation formula of specific emissivity is: specific emissivity = 1 - (TCM ^ 4 – TSM ^ 4) / (Tcha ^ 4 – tsky ^ 4). This data can provide basic data for remote sensing inversion algorithm of surface temperature and surface radiation energy balance.

Data source description

The naming rule of each observation point is in the form of a3-9, which is expressed as the No. 9 sampling point of a rou quadrat 3. Each center point is randomly measured twice.

Data processing method

The horizontal surface shall be selected as far as possible to ensure that the measurement angle is 45 degrees; And in strict accordance with (1) adding mirrors without capping (tsky)（ 2) Mirror capping (Tcha)（ 3) Complete one measurement in the order of no mirror and no cover (TSM) and (4) no mirror and no cover (TCM). To reduce the error, read 4 numbers for each observation.

Data quality description

Good data quality

Base information

collect time:2008/03/14-2008/03/15

collect place:Heihe River Basin, upstream cold region, hydrological test area

data size:81.1 MiB

data format:excel

Coordinate system: WGS84

Keywords

Theme:Portable specific emissivity meter, specific emissivity, thermal imager, surface radiation temperature, handheld infrared thermometer, airborne ground remote sensing
	Subject:physical geography 
	Time:2008
	Place:Heihe River Basin,  upstream cold region,  hydrological test area

Space scope

|  | north： 38.97 |  |
| --- | --- | --- |
| west：100.59 |  | east：100.44 |
|  | south： 38.04 |  |

Service information

contacts:Min Yufang

phone:0931-4967596

mailbox:ncdc@lzb.ac.cn

Contact information

contributors:JIN Rui

author:JIN Rui

manager:JIN Rui

Project information

Citations and annotations

In order to protect the rights and interests of the platform's scientific and technological resources, expand the services of the platform center, and enhance the application potential of scientific and technological resources,resource users are requested to mark and quote the research results (including published papers, treatises, data products, unpublished research reports, data products, etc.) generated by the use of resources in the following manner.

For the achievements published in English, please refer to the following specifications：

The dataset is provided by National Cryosphere Desert Data Center(https://www.ncdc.ac.cn).

Data reference:

JIN Rui. Heihe River Integrated Remote Sensing joint experiment: thermal infrared specific emissivity observation data set in ARW intensive observation area (2008). National Cryosphere Desert Data Center(http://www.ncdc.ac.cn), 2022. https://cstr.cn/CSTR:11738.11.ncdc.NIEER.2021.1911.