The SRTM DEM dataset in the Black River Basin

DOI：10.12072/ncdc.westdc.db3672.2023

CSTR：11738.11.ncdc.Westdc.2020.462

Share type:Login to Access

Category:Basic Geography

Datasets description

SRTM (Shuttle Radar topology mission) is a global 3D graphics data project jointly established by NASA and National Geospatial Intelligence Agency (NGA). In February 2000, the SRTM system of the U.S. space shuttle Endeavour collected radar image data between 60 ° N and 57 ° s, and obtained radar image data covering more than 80% of the world's land surface. After more than two years of processing, a digital terrain elevation model is made. This data set includes two kinds of data: SRTM sub map and mosaic map of Heihe River Basin, in which the sub map is the data of SRTM version 4, which is compiled by cgiar-csi (International Tropical Agriculture Center, Http://srtm.csi.cgiar.org/) processing, compared with the previous versions, has greatly improved: 1) a large number of interpolation algorithms are used, 2) more auxiliary DEM data are used to fill in the white spots and blank areas, 3) compared with the data of the third version, the image element is offset by one-half. Mosaic is obtained by splicing on the basis of sub graphs. The sub maps include srtm～04, SRTM ᦉ 05, SRTM ᦉ 04, SRTM ᦉ 57 ᦉ 054 maps. The data are 16 digit values representing the elevation value (- / + / 32767 m), the maximum positive elevation is 9000 m, and the negative elevation is 12000 m below sea level. Empty data is identified with - 32767. A file is divided into 24 lines (- 60 to 60 degrees) and 72 columns (- 180 to 180 degrees) every 5 degrees of longitude and latitude.

Data source description

Two kinds of data of SRTM and mosaic map in Heihe River Basin

Data processing method

Interpolation algorithm

Data quality description

Good data quality

Base information

collect time:2000/01/01-2000/12/31

collect place:Heihe River Basin

data size:217.9 MiB

data format:Raster data

Coordinate system: WGS84

Projection
       +proj=longlat +datum=WGS84 +no_defs

Keywords

Theme:Digital elevation model, Mosaic, SRTM, DEM
	Subject:physical geography 
	Time:2000
	Place:Heihe Basin

Space scope

|  | north： 43.27 |  |
| --- | --- | --- |
| west： 96.07 |  | east：104.20 |
|  | south： 37.70 |  |

Service information

contacts:Heihe project data management center

phone:0931-4967287

mailbox:westdc@lzb.ac.cn

Contact information

contributors:Heihe project data management center

author:Heihe project data management center

manager:Heihe project data management center

Project information

Citations and annotations

In order to protect the rights and interests of the platform's scientific and technological resources, expand the services of the platform center, and enhance the application potential of scientific and technological resources,resource users are requested to mark and quote the research results (including published papers, treatises, data products, unpublished research reports, data products, etc.) generated by the use of resources in the following manner.

For the achievements published in English, please refer to the following specifications：

The dataset is provided by National Cryosphere Desert Data Center(https://www.ncdc.ac.cn).

Data reference:

Heihe project data management center. The SRTM DEM dataset in the Black River Basin. National Cryosphere Desert Data Center(http://www.ncdc.ac.cn), 2020. https://cstr.cn/CSTR:11738.11.ncdc.Westdc.2020.462.

Article citation:

Jarvis, A., H.I. Reuter, A. Nelson, E. Guevara, 2008, Hole-filled SRTM for the globe Version 4, available from the CGIAR-CSI SRTM 90m Database: http://srtm.csi.cgiar.org.