Precipitation observation data set of Qiaogou watershed in the first sub-region of Loess Hills and Gullies area from 1986 to 1990

DOI：10.12072/ncdc.HWSDZhan.db0020.2020

CSTR：11738.11.ncdc.HWSDZhan.2020.80

Share type:Apply to Access

Category:Soil and Water Conservation

Datasets description

Data source description

Instrument continuous observation and manual timing observation

Data processing method

Precipitation observation instruments include dsj2 siphon rain gauge and SM1 ordinary rain gauge with diameter of 20cm. Dsj2 self recording rain gauge adopts 24 segment system for flood season observation, SM1 ordinary rain gauge adopts 8:00 and 20:00 two-stage system for annual observation. In case of failure of self recording rain gauge, observation data of ordinary rain gauge are used for supplement. It mainly observes the beginning and ending time of precipitation and statistics of precipitation. When the intermittent time of precipitation is more than 15min, it is observed as two precipitation before and after the interval. When the intermittent time is less than 15min, it is recorded as a precipitation observation. If a precipitation crosses 8:00 and 20:00, it should be measured at 8:00 and 20:00.

Data quality description

The instrument calibration shall be carried out before the start of annual observation of rain gauge. In the process of data entry, there are four levels of manual verification, including tabulation, primary proofreading, rechecking and auditing, so as to ensure the data quality.

Base information

collect time:1986/01/01-1990/12/31

collect place:Qiaogou watershed, Suide County, Yulin City, Shanxi Province

data size:496.7 KiB

data format:xls，doc，pdf

Coordinate system: WGS84

Keywords

Theme:Daily precipitation
	Subject:
	Time:1986 1987 1988 1989 1990
	Place:Qiaogou,  Yulin,  Suide,  Shaanxi Province

Space scope

|  | north： 37.50 |  |
| --- | --- | --- |
| west：110.29 |  | east：110.30 |
|  | south： 37.49 |  |

Service information

contacts:Min Yu Fang

phone:0931-4967596

mailbox:ncdc@lzb.ac.cn

Contact information

contributors:Suide administration and Supervision Bureau of water and soil conservation of the Yellow River (Suide scientific experimental station of water and soil conservation)

author:LEI Xin

manager:GUO Yumei , Suide administration and Supervision Bureau of water and soil conservation of the Yellow River (Suide scientific experimental station of water and soil conservation)

Project information

number:126201340005150007，
      name:

Citations and annotations

In order to protect the rights and interests of the platform's scientific and technological resources, expand the services of the platform center, and enhance the application potential of scientific and technological resources,resource users are requested to mark and quote the research results (including published papers, treatises, data products, unpublished research reports, data products, etc.) generated by the use of resources in the following manner.

For the achievements published in English, please refer to the following specifications：

The dataset is provided by National Cryosphere Desert Data Center(https://www.ncdc.ac.cn).

Data reference:

Suide administration and Supervision Bureau of water and soil conservation of the Yellow River (Suide scientific experimental station of water and soil conservation). Precipitation observation data set of Qiaogou watershed in the first sub-region of Loess Hills and Gullies area from 1986 to 1990. National Cryosphere Desert Data Center(http://www.ncdc.ac.cn), 2020. https://cstr.cn/CSTR:11738.11.ncdc.HWSDZhan.2020.80.