Observation data set of runoff and sediment in Jiuyuangou watershed, the first sub region of Loess Hilly and gully region from 1964 to 1979

DOI：10.12072/ncdc.HWSDZhan.db0046.2020

CSTR：11738.11.ncdc.HWSDZhan.2020.69

Share type:Login to Access

Category:Soil and Water Conservation

Datasets description

This data set is the runoff and sediment observation data of Jiuyuangou River Basin from 1964 to 1979. Through the water level observation and sediment sampling of runoff station section, the runoff and sediment amount during runoff generation are calculated. The data set includes the measured discharge results table, flood hydrological elements extraction table, successive flood test results table, daily average discharge table, daily average sediment concentration table and daily average sediment transport rate table of Jiuyuangou watershed.

Data source description

Artificial timing observation

Data processing method

1. The water level observation method of runoff station is as follows: read the water gauge number of the flow measurement structure on the flow measurement section, and observe once a day at 8:00 in non flood season; observe twice at 8:00 and 20:00 when the daily water level changes slowly in case of rainfall; and observe four times at 2:00, 8:00, 14:00 and 20:00 when the daily water level changes greatly or appears slow peak and valley. When the water level changes violently in flood season, the observation shall be made in multiples of 6 minutes. 2. The flow measurement mode of the ditch diameter flow station: the flow is measured by the delta channel when the water level is flat; when the water level is higher than the delta channel, the flow measurement is conducted by the buoy method in the rectangular spillway, and the buoy coefficient is the buoy test results of huangweizizhou runoff test station. 3. The flow measurement method of wangmaozhuang runoff station: the calibrated natural section observation by manual arrangement is adopted, and the calibrated flow formula is used for flow calculation. 4. Sediment sampling and treatment: the midstream buoy method is adopted for sediment sampling at the flow measurement section. The number of sediment sampling depends on the number of flood water level changes. The sampling times are increased to control the change of sediment concentration during flood peak. The sample was treated by displacement method.

Data quality description

Manual observation is carried out by professional and technical personnel; four levels of manual verification are carried out in the process of data input, which are tabulation, initial calibration, re calibration and audit, so as to ensure the data quality.

Base information

collect time:1964/01/01-1979/12/31

collect place:Jiuyuangou watershed, Suide County, Yulin City, Shaanxi Province

altitude:2.0m- Nonem

data size:456.7 KiB

data format:Excel&Word

Coordinate system: WGS84

Keywords

Theme:Runoff modulus, runoff coefficient, sediment transport rate, water level, erosion modulus, discharge, sediment concentration, runoff depth, runoff, soil loss
	Subject:Hydrometeorology 
	Time:1964 1965 1966 1967 1968 1969 1970 1971 1972 1973 1974 1975 1976 1977 1978 1979
	Place:Jiuyuangou,  Suide,  Yulin,  Shaanxi Province

Space scope

|  | north： 37.62 |  |
| --- | --- | --- |
| west：110.26 |  | east：110.39 |
|  | south： 37.54 |  |

Service information

contacts:Min Yu Fang

phone:093149675976

mailbox:ncdc@lzb.ac.cn

Contact information

contributors:Suide administration and Supervision Bureau of water and soil conservation of the Yellow River (Suide scientific experimental station of water and soil conservation)

author:LEI Xin ,  LiuSijun

manager:GUO Yumei , Suide administration and Supervision Bureau of water and soil conservation of the Yellow River (Suide scientific experimental station of water and soil conservation)

Project information

Citations and annotations

In order to protect the rights and interests of the platform's scientific and technological resources, expand the services of the platform center, and enhance the application potential of scientific and technological resources,resource users are requested to mark and quote the research results (including published papers, treatises, data products, unpublished research reports, data products, etc.) generated by the use of resources in the following manner.

For the achievements published in English, please refer to the following specifications：

The dataset is provided by National Cryosphere Desert Data Center(https://www.ncdc.ac.cn).

Data reference:

Suide administration and Supervision Bureau of water and soil conservation of the Yellow River (Suide scientific experimental station of water and soil conservation). Observation data set of runoff and sediment in Jiuyuangou watershed, the first sub region of Loess Hilly and gully region from 1964 to 1979. National Cryosphere Desert Data Center(http://www.ncdc.ac.cn), 2020. https://cstr.cn/CSTR:11738.11.ncdc.HWSDZhan.2020.69.