Heihe River eco hydrological remote sensing test: observation data set of water surface temperature of Erba reservoir in the middle reaches of Heihe River Basin (2012)

DOI：10.12072/ncdc.nieer.db3501.2023

CSTR：11738.11.ncdc.NIEER.2021.1879

Share type:Apply to Access

Category:Remote Sensing

Datasets description

The infrared temperature and water surface temperature observation system of Erba reservoir is set up to provide water surface temperature data for aviation TASI, widas and L-band flights. Observation site: the Erba reservoir, 14km east of Zhangye City and next to Erba village, Jiantan Town, Ganzhou District, is selected as the observation point. Coordinates of the observation point: 38 ° 54'57.14 "n, 100 ° 36'57.39". Measuring instrument: the observation contents include a si-111 infrared temperature probe perpendicular to the water surface and a si-111 infrared temperature probe observing the sky at a zenith angle of 35 degrees due south (the instrument was newly purchased in 2012, the default surface emissivity was 1 when it was delivered from the factory, and blackbody calibration was not carried out during the test). Two 0cm109ss-l temperature sensors floating below the water surface, data acquisition is Campell cr-1000, automatic acquisition, GPRS wireless transmission to the comprehensive collection system. The probe frame is 3M high and the offshore distance is 3.4m. Measurement time: the instrument starts normal observation from May 27, 2012 to September 27, 2012, continuously observes for 24 hours, records data once in 5 seconds, and outputs two groups of values in 5 seconds and 1 minute. Data content: vertical water surface temperature (tart)_ Sur, without correction of surface specific emissivity and background temperature), sky temperature (tart)_ ATM, without sky background specific emissivity correction), water surface 0cm directly measured temperature (water t)_ 1，WaterT_ 2）。 The data is finally stored as an independent text file for one day. The data naming method is: data format + observation point name + data sampling time + date + time. Dat. For detailed data header information, see the data header description in the data

Data source description

14km east of Zhangye City, the Erba reservoir next to Erba village, Jiantan Town, Ganzhou district is used as the observation point.

Data processing method

It includes a si-111 infrared temperature probe perpendicular to the water surface and a si-111 infrared temperature probe for sky observation with a zenith angle of 35 degrees due to the South (the instrument was newly purchased in 2012, the default surface emissivity was 1 when it was delivered from the factory, and blackbody calibration was not carried out during the test). Two 0cm109ss-l temperature sensors floating below the water surface, data acquisition is Campell cr-1000, automatic acquisition, GPRS wireless transmission to the comprehensive collection system. The probe frame is 3M high and the offshore distance is 3.4m

Data quality description

Vertical water surface temperature (tart)_ Sur, without correction of surface specific emissivity and background temperature), sky temperature (tart)_ ATM, without sky background specific emissivity correction), water surface 0cm directly measured temperature (water t)_ 1，WaterT_ 2）。

Base information

collect time:2012/06/04-2012/10/05

collect place:Heihe River Basin, artificial oasis experimental area in the middle reaches, Erba reservoir

data size:67.8 MiB

data format:

Coordinate system: WGS84

Keywords

Theme:Water surface temperature, water body, radiation calibration, calibration observation, surface radiation temperature, water area, land use, surface radiation characteristics
	Subject:physical geography 
	Time:2012
	Place:Experimental area of artificial oasis in the middle reaches,  Erba reservoir,  Heihe River Basin

Space scope

|  | north： 38.92 |  |
| --- | --- | --- |
| west：100.62 |  | east：100.62 |
|  | south： 38.92 |  |

Service information

contacts:Min Yufang

phone:0931-4967596

mailbox:ncdc@lzb.ac.cn

Contact information

contributors:Ma mingguo

author:Ma mingguo

manager:Ma mingguo

Project information

number:91125004，
      name:

Citations and annotations

In order to protect the rights and interests of the platform's scientific and technological resources, expand the services of the platform center, and enhance the application potential of scientific and technological resources,resource users are requested to mark and quote the research results (including published papers, treatises, data products, unpublished research reports, data products, etc.) generated by the use of resources in the following manner.

For the achievements published in English, please refer to the following specifications：

The dataset is provided by National Cryosphere Desert Data Center(https://www.ncdc.ac.cn).

Data reference:

Ma mingguo. Heihe River eco hydrological remote sensing test: observation data set of water surface temperature of Erba reservoir in the middle reaches of Heihe River Basin (2012). National Cryosphere Desert Data Center(http://www.ncdc.ac.cn), 2021. https://cstr.cn/CSTR:11738.11.ncdc.NIEER.2021.1879.

Article citation:

Xin Li, Shaomin Liu, Qin Xiao, Mingguo Ma, Rui Jin, Tao Che, Weizhen Wang, Xiaoli Hu, Ziwei Xu, Jianguang Wen, Liangxu Wang. A multiscale dataset for understanding complex eco-hydrological processes in a heterogeneous oasis system.[J]. Scientific data, 2017, 4.