Surface elevation changes of glaciers in the Tuomu'er Peak area of Tianshan Mountains (2000-2010)

DOI：10.12072/ncdc.tianshan.db7103.2026

CSTR：11738.11.NCDC.TIANSHAN.DB7103.2026

Share type:Apply to Access

Category:Hydrology

Datasets description

This dataset focuses on the glaciers in the Tuomu'er Peak area of the Tianshan Mountains. Using SRTM V3 and FABDEM V1-2 remote sensing data products, the surface elevation changes of glaciers are obtained through difference calculation. The unit is meters (m). The dataset is mainly used for extracting data from remote sensing images, and only represents the elevation changes of glacier surfaces at the time of remote sensing image acquisition, from 2000 to 2010.

Data source description

SRTM V3: jointly measured by NASA and NIMA on February 11, 2000. This data product has been publicly released since 2003 and has undergone multiple revisions. The current revised version of the data is V3. This version uses more advanced algorithms and integrates multi-source data to fill the gaps in the original SRTM data (mainly caused by water bodies and radar shadows). These auxiliary data include ASTER GDEM v2, USGS GMTED2010, data from the TanDEM-X mission mentioned earlier, and other regional terrain data. 90 meter resolution DEM.     FABDEM V1-2: The full name is Forest And Buildings removed Copernicus DEM. It is a global digital elevation model (DEM) that removes the height of trees and buildings. Based on the Copernicus DEM (GLO-30) of the European Space Agency (ESA), the data itself comes from the TanDEM-X radar satellite mission and has high vertical accuracy. Using advanced machine learning algorithms (primarily convolutional neural networks) and auxiliary data (such as global forest height maps and building footprint data), identify and subtract forest canopy and building heights worldwide. 30 meter resolution.

Data processing method

After conducting accuracy checks on DEMs from different periods, the cosine relationship between slope direction and elevation difference is used for plane and elevation correction; Then, by utilizing the relationship between maximum curvature and elevation difference, the errors caused by different resolutions are reduced, and the two phases of DEM are registered and the relative accuracy is evaluated to obtain the glacier elevation changes in different periods.

Data quality description

The data quality is good.

Base information

collect time:2000/01/01-2010/12/31

collect place:Tian Shan, Kuitun River Basin

altitude:2900.0m- 7400.0m

data size:502.4 MiB

data format:tif

Coordinate system: WGS84

Projection
       UTM

Keywords

Theme:Glacier, surface elevation, Tianshan Tuomu'er Peak area
	Subject:Earth Science 
	Time:2000 2001 2002 2003 2004 2005 2006 2007 2008 2009 2010
	Place:Tianshan Tuomu'er Peak area

Space scope

|  | north： 42.30 |  |
| --- | --- | --- |
| west： 79.50 |  | east： 80.50 |
|  | south： 41.00 |  |

Service information

contacts:Min Yufang

phone:09314967596

mailbox:ncdc@lzb.ac.cn

Contact information

contributors:

author:

manager:

Project information

number:2023NCDC008，
      name:Compilation and Integration of the Tianshan Glaciers Data Resource System

Citations and annotations

In order to protect the rights and interests of the platform's scientific and technological resources, expand the services of the platform center, and enhance the application potential of scientific and technological resources,resource users are requested to mark and quote the research results (including published papers, treatises, data products, unpublished research reports, data products, etc.) generated by the use of resources in the following manner.

For the achievements published in English, please refer to the following specifications：

The dataset is provided by National Cryosphere Desert Data Center(https://www.ncdc.ac.cn).

Data reference:

. Surface elevation changes of glaciers in the Tuomu'er Peak area of Tianshan Mountains (2000-2010). National Cryosphere Desert Data Center(http://www.ncdc.ac.cn), 2026. https://cstr.cn/CSTR:11738.11.NCDC.TIANSHAN.DB7103.2026.