Heihe River Integrated Remote Sensing joint experiment: ground-based microwave radiometer and surface observation data set in ARW intensive observation area (2008)

DOI：10.12072/ncdc.nieer.db2993.2023

CSTR：11738.11.ncdc.NIEER.2021.1845

Share type:Login to Access

Category:Remote Sensing

Datasets description

This data set includes 7 files, which are: S-band ground-based microwave radiometer observation data, C-band ground-based microwave radiometer observation data, K-band ground-based microwave radiometer observation data, Ka band ground-based microwave radiometer observation data, soil temperature automatic observation data, temperature manual observation data, soil moisture automatic observation data. The moisture data and temperature data are stored in Excel.

Data source description

During April 5-8, 2008, continuous observation with multi frequency, multi polarization and multi angle ground-based microwave radiometer was carried out in the leveling bare soil of aryo encryption observation area, including S-band ground-based microwave radiometer observation (April 6-8), C-band ground-based microwave radiometer observation (April 7-8, 2008), K-band ground-based microwave radiometer observation (April 5-8) and Ka band ground-based microwave radiometer observation (April 5). The main observation target is the influence of surface freezing and thawing state on microwave brightness temperature

Data processing method

The site is flat bare soil, with an initial dry water content of about 14%, and then watered evenly manually, with a water content of about 30%. The Soil Hydrothermal observation field is arranged in the east of the microwave radiometer observation field, including 5cm automatic soil temperature observation (10 minute observation step); Manual observation of 5cm, 10cm, 20cm and 30cm soil temperature (needle thermometer with 1-hour observation step); Automatic observation of 5cm, 10cm, 20cm and 30cm soil moisture (10 minute observation step). This data can provide basic data for the development of Microwave Radiative transfer forward model and microwave remote sensing inversion model of frozen / thawed soil

Data quality description

Good data quality

Base information

collect time:2008/04/05-2008/04/08

collect place:Heihe River Basin, a rou densification observation station, upstream cold region hydrological Experiment Station

data size:10.2 MiB

data format:dat,txt

Coordinate system: WGS84

Keywords

Theme:Heihe River Basin, S-band ground-based microwave radiometer observation data, K-band ground-based microwave radiometer observation data, Ka band ground-based microwave radiation, C-band ground-based microwave radiometer observation data, soil moisture, soil temperature, ground verification information
	Subject:physical geography 
	Time:2008
	Place:Heihe River Basin,  upstream cold region,  hydrological test area

Space scope

|  | north： 38.97 |  |
| --- | --- | --- |
| west：100.59 |  | east：100.44 |
|  | south： 38.04 |  |

Service information

contacts:Min Yufang

phone:0931-4967596

mailbox:ncdc@lzb.ac.cn

Contact information

contributors:JIN Rui,  Che Tao,  HAO Xiaohua

author:JIN Rui

manager:JIN Rui

Project information

Citations and annotations

In order to protect the rights and interests of the platform's scientific and technological resources, expand the services of the platform center, and enhance the application potential of scientific and technological resources,resource users are requested to mark and quote the research results (including published papers, treatises, data products, unpublished research reports, data products, etc.) generated by the use of resources in the following manner.

For the achievements published in English, please refer to the following specifications：

The dataset is provided by National Cryosphere Desert Data Center(https://www.ncdc.ac.cn).

Data reference:

JIN Rui, Che Tao, HAO Xiaohua . Heihe River Integrated Remote Sensing joint experiment: ground-based microwave radiometer and surface observation data set in ARW intensive observation area (2008). National Cryosphere Desert Data Center(http://www.ncdc.ac.cn), 2021. https://cstr.cn/CSTR:11738.11.ncdc.NIEER.2021.1845.