Hourly observation data from a 10 meter meteorological tower on a grassland at an altitude of 2575 meters in the Dayekou watershed of Qilian Mountains (December 2015 December 2020)

DOI：10.12072/ncdc.qilian_wrcf.db7083.2026

CSTR：11738.11.NCDC.QILIAN_WRCF.DB7083.2026

Share type:Apply to Access

Category:Meteorology

Datasets description

This dataset consists of 1-hour time scale conventional meteorological observation data from a 2575 meter meteorological tower in the Dayekou watershed of the Qilian Mountains from December 2015 to December 2020. The coordinates are N 38 ° 34 ′ 1.3 ″ and E 100 ° 17 ′ 18.3 ″; Altitude: 2575m; observation elements include: temperature (℃) at 2m and 10m, relative humidity (%) and water vapor pressure (Kpa) at 2m and 10m, wind speed (m/s) and direction (°) at 2m and 10m, four component radiation (W/m ^ 2), sunshine hours (h), atmospheric pressure (hpa), precipitation (mm), soil heat flux (W/m ^ 2), surface temperature at 0cm (℃), soil temperature (℃), soil moisture (m ^ 3/m ^ 3), conductivity (dS/m), and soil three parameter sensors are deployed at depths of 10cm, 20cm, 30cm, 40cm, 60cm, and 80cm. The automatic observation of precipitation is normal from May to October every year, but the Qilian Mountains are in the freezing season from November to April of the following year, and precipitation cannot be automatically observed. This data is obtained through 10 minute data collection within 1 hour, where the sunshine hours and rainfall are the sum of the 10 minute data, and the rest are the average values of the 10 minute data.

Data source description

Automatic collection; Data collector and sensor models: Campbell CR6 data collector, HMP155 air temperature and humidity sensor, WindSonic two-dimensional ultrasonic wind speed and direction sensor, NR01 four component radiation sensor, SD4 sunshine hours sensor, PTB110 air pressure sensor, TR-525MM tipping bucket rain gauge, SI-111-SS surface infrared temperature sensor (installed in October 2018), CSS655 soil temperature, humidity, and salinity three parameter sensor.

Data processing method

This data is based on the collected raw measurement data (10 minute data table) and is statistically analyzed after removing outliers. The sunshine hours and precipitation are the sum of 10 minute data within 1 hour, while the rest are the average of 10 minute data within 1 hour. The lack of data in certain time periods is due to equipment malfunctions or after removing outliers.

Data quality description

The data is obtained by statistical analysis based on the original measurement data after removing outliers, and the data quality is excellent.

Base information

collect time:2015/12/01-2020/12/31

collect place:Meteorological Observation Field of Qilian Mountain Forest Ecological Station in Gansu Province

altitude:2575.0m- 2575.0m

data size:12.6 MiB

data format:.xlsx

Coordinate system: WGS84

Keywords

Theme:Meteorology, temperature, humidity, air pressure, wind speed, wind direction, soil temperature, soil moisture, soil heat flux, precipitation, radiation, sunshine hours
	Subject:Atmospheric Science , Hydrometeorology 
	Time:2015 2016 2017 2018 2019 2020
	Place:Qilian Mountains,  Dayekou

Space scope

|  | north： 38.57 |  |
| --- | --- | --- |
| west：100.29 |  | east：100.29 |
|  | south： 38.57 |  |

Service information

contacts:Li Hongxing

phone:0931-4967592

mailbox:ncdc@lzb.ac.cn

Contact information

contributors:WANG Rongxin,  JING Wenmao

author:WANG Rongxin

manager:WANG Rongxin , LI Hongxing

Project information

number:2024NCDC005，
      name:

Citations and annotations

In order to protect the rights and interests of the platform's scientific and technological resources, expand the services of the platform center, and enhance the application potential of scientific and technological resources,resource users are requested to mark and quote the research results (including published papers, treatises, data products, unpublished research reports, data products, etc.) generated by the use of resources in the following manner.

For the achievements published in English, please refer to the following specifications：

The dataset is provided by National Cryosphere Desert Data Center(https://www.ncdc.ac.cn).

Data reference:

WANG Rongxin, JING Wenmao. Hourly observation data from a 10 meter meteorological tower on a grassland at an altitude of 2575 meters in the Dayekou watershed of Qilian Mountains (December 2015 December 2020). National Cryosphere Desert Data Center(http://www.ncdc.ac.cn), 2026. https://cstr.cn/CSTR:11738.11.NCDC.QILIAN_WRCF.DB7083.2026.