Data set of glacier surface elevation changes in the main peak area of Tanggula Mountain, Nianqing, 1970-2014

DOI：10.12072/ncdc.DIMaC.db0076.2020

CSTR：11738.11.ncdc.DIMaC.2020.128

Share type:Apply to Access

Category:Glacier

Datasets description

This data set aims at the main peak area of Nianqing Tanggula Mountain, including the main peak glacier and surrounding areas of Nianqing Tanggula Mountain. Using the DEM extracted from topographic map, SRTM (2000) and remote sensing image, the surface elevation change of glacial center is obtained by difference calculation. Only for the change of glacier surface elevation in the main peak area of Tanggula Mountain in Nianqing from 1970 to 2014, it is not supplemented and improved, and it is fully open sharing. The dataset contains two wavebands, elevationchange1970-2000 and elevationchange2000-2014. Field measurement unit: meter (m). The data set mainly includes remote sensing image extraction data, which only represents the glacier surface elevation change at the acquisition time of remote sensing image. The time is 1970-2000 and 2000-2014.

Data source description

This data set is mainly interpretation data, and the main data sources are topographic map DEM, SRTM (2000) and remote sensing image DEM.

Data processing method

Using DEM from topographic map, SRTM (2000) and remote sensing image, the elevation change of glacier surface in the center of glaciation was obtained by difference calculation.

Data quality description

After checking the accuracy of DEM in different periods, we use the cosine relationship between slope aspect and elevation difference to carry out plane and elevation correction; then we use the relationship between maximum curvature and elevation difference to reduce the error caused by different resolutions, carry out registration and relative accuracy evaluation on the two periods of DEM, and get the glacier elevation changes in different periods.

Base information

collect time:1970/01/01-2014/12/31

collect place:

data size:24.7 MiB

data format:

Coordinate system: WGS84

Keywords

Theme:Ice elevation change, Nianqingtang Gula Mountain main peak, DEM
	Subject:physical geography 
	Time:1970-2014
	Place:Nianqingtang Gula Mountain main peak

Space scope

|  | north： 30.79 |  |
| --- | --- | --- |
| west： 89.96 |  | east： 91.03 |
|  | south： 29.86 |  |

Service information

contacts:Li Hongxing

phone:0931-4967592

mailbox:lihongxing@lzb.ac.cn

Contact information

contributors:LIU Shiyin,  WU Kunpeng

author:WU Kunpeng

manager:WU Kunpeng , LIU Shiyin

Project information

number:2013FY111400，
      name:

Citations and annotations

In order to protect the rights and interests of the platform's scientific and technological resources, expand the services of the platform center, and enhance the application potential of scientific and technological resources,resource users are requested to mark and quote the research results (including published papers, treatises, data products, unpublished research reports, data products, etc.) generated by the use of resources in the following manner.

For the achievements published in English, please refer to the following specifications：

The dataset is provided by National Cryosphere Desert Data Center(https://www.ncdc.ac.cn).

Data reference:

LIU Shiyin, WU Kunpeng. Data set of glacier surface elevation changes in the main peak area of Tanggula Mountain, Nianqing, 1970-2014. National Cryosphere Desert Data Center(http://www.ncdc.ac.cn), 2021. https://cstr.cn/CSTR:11738.11.ncdc.DIMaC.2020.128.