Observation dataset of precipitation in Jiuyuangou watershed, the first sub area of loess hilly and gully region in 2023

DOI：10.12072/ncdc.hwsdzhan.db6584.2024

CSTR：11738.11.NCDC.HWSDZHAN.DB6584.2024

Share type:Apply to Access

Category:Soil and Water Conservation

Datasets description

This dataset contains precipitation observation data from the Jiuyuangou watershed rainfall station in 2023. The Jiuyuangou watershed is located in Suide County, Yulin City, Shaanxi Province, and is a first level tributary of the Wuding River. The rainfall station focuses on observing the precipitation during the flood season (June September) on the basis of observing the annual precipitation. Nine rainfall stations have been set up in the Jiuyuangou watershed, including Goukou, Maliangou, Zhaojiagou, Wujiapan, Weijiayan, Heijiagou, Wangjiagou, Wangmaozhuang, and Caijiagou. Use DSJ2 siphon type self recording rain gauge and SM1 type ordinary rain gauge with a diameter of 20cm for parallel observation, and replace them with ordinary rain gauges when the self recording rain gauge malfunctions. The data includes the layout map of the watershed station network, daily precipitation table, precipitation extraction table, and maximum precipitation table for each period. For the basic information of the rainfall station in the Jiuyuangou watershed, please refer to the "Description of the Jiuyuangou Watershed in 2023"

Data source description

Continuous observation by instrument and regular observation manually

Data processing method

<p>There are SM1 rain gauge with diameter of 20cm and dsj2 siphon self recording rain gauge. SM1 adopts 8:00 and 20:00 two-stage system for annual observation, mainly observing the beginning and ending time of precipitation, and counting the precipitation. When the intermittent time of precipitation is more than 15 min, it is observed as two precipitation before and after the interval. When the intermittent time is less than 15 min, it is regarded as a precipitation observation record. If a precipitation crosses 8:00 and 20:00, it should be measured at 8:00 and 20:00. Dsj2 adopts 24 segment system for flood season observation. In case of failure of self recording rain gauge, the observation data of ordinary rain gauge are supplemented. </p>

Data quality description

The recording rain gauge shall be calibrated before the start of observation every year. In the process of data entry, there are four levels of manual verification: tabulation, initial verification, re verification, and audit to ensure data quality

Base information

collect time:2023/01/01-2023/12/31

collect place:Jiuyuangou Valley, Jiuyuangou Township, Suide County, Yulin City, Shaanxi Province

data size:417.7 KiB

data format:

Coordinate system: WGS84

Keywords

Theme:Extract of precipitation, Maximum precipitation, Precipitation intensity, Period precipitation
	Subject:
	Time:2023
	Place:Suide,  Yulin,  Jiuyuangou,  Shaanxi Province

Space scope

|  | north： 37.62 |  |
| --- | --- | --- |
| west：110.26 |  | east：110.39 |
|  | south： 37.54 |  |

Service information

contacts:Min Yu Fang

phone:09314967597

mailbox:myf@lzb.ac.cn

Contact information

contributors:Suide administration and Supervision Bureau of water and soil conservation of the Yellow River (Suide scientific experimental station of water and soil conservation)

author: ,

manager:Suide administration and Supervision Bureau of water and soil conservation of the Yellow River (Suide scientific experimental station of water and soil conservation) , LEI Xin

Project information

Citations and annotations

In order to protect the rights and interests of the platform's scientific and technological resources, expand the services of the platform center, and enhance the application potential of scientific and technological resources,resource users are requested to mark and quote the research results (including published papers, treatises, data products, unpublished research reports, data products, etc.) generated by the use of resources in the following manner.

For the achievements published in English, please refer to the following specifications：

The dataset is provided by National Cryosphere Desert Data Center(https://www.ncdc.ac.cn).

Data reference:

Suide administration and Supervision Bureau of water and soil conservation of the Yellow River (Suide scientific experimental station of water and soil conservation). Observation dataset of precipitation in Jiuyuangou watershed, the first sub area of loess hilly and gully region in 2023. National Cryosphere Desert Data Center(http://www.ncdc.ac.cn), 2024. https://cstr.cn/CSTR:11738.11.NCDC.HWSDZHAN.DB6584.2024.