Joint experiment of Heihe comprehensive remote sensing: observation data set of solar spectrophotometer in dense observation area of Yingke oasis and huazhaizi desert

DOI：10.12072/ncdc.NIEER.db1787.2022

CSTR：11738.11.ncdc.NIEER.2021.1807

Share type:Login to Access

Category:Remote Sensing

Datasets description

Widas (wide angle infrared dual-mode line / area array) was obtained by CE318 solar spectrophotometer in Yingke oasis and huazhaizi desert intensive observation area Scanner, omis-2, TM, aster, Chris and Hyperion, as well as conventional observation of atmospheric parameters provide important atmospheric parameters for atmospheric correction of various remote sensing images and ground measurement data. The original data is stored in CE318 special file format. K7 and can be opened by astpwin software. The preprocessing file includes the retrieval of optical thickness, Rayleigh scattering, aerosol optical thickness, horizontal visibility and near surface atmospheric temperature from the original data, as well as the solar azimuth, zenith angle, sun earth distance correction factor and atmospheric column mass involved in the calculation. Data preprocessing result file includes two parts: data processing description and data processing result. The former introduces the basic principle of data processing, while the latter gives the results of data processing, including "geometric position and total optical thickness of each channel" and "Rayleigh scattering and aerosol optical thickness of each channel".

Data source description

The infrared temperature data are calibrated; In the original record, the number of infrared thermometer in sample a area is Feng Lei -#3, which lacks the corresponding calibration coefficient, so the infrared temperature calibration cannot be carried out; There is no data record for flight belt 10 in sample area B; The end time is not recorded in line7 of flight belt 8 data and line5 of 120m quadrat data in sample area C.   CE318 solar spectrophotometer can inverse the optical thickness, Rayleigh scattering and aerosol optical thickness of non water vapor channel through the direct solar radiation measurement data. The water vapor content of atmospheric gas column can be obtained from the 936nm measurement data of water vapor channel. According to the needs, the data can obtain various parameters at 550nm, so as to obtain horizontal visibility with the help of software such as MODTRAN or 6S   Test instrument: one CE318 of Beijing Normal University and one CE318 of Institute of remote sensing of Chinese Academy of sciences are adopted. Among them, CE318 of Beijing Normal University can provide the optical thickness of five bands: 1020nm, 936nm, 870nm, 670nm and 440nm, and the water vapor content of atmospheric column can be retrieved by using 936nm measurement data; CE318, Institute of remote sensing, Chinese Academy of Sciences, can provide the optical thickness of 9 bands: 1640nm, 1020nm, 936nm, 870nm, 670nm, 550nm, 440nm, 380nm and 340nm, and can retrieve the water vapor content of the atmospheric column using 936nm measurement data   Measurement time: the two instruments carried out 24 observations in 15 days in Yingke oasis and huazhaizi desert intensive observation area and Zhangye intensive observation area, respectively: 2008-05-202008-05-232008-05-252008-05-272008-06-042008-06-062008-06-162008-06-202008-06-222008-06-232008-06-272008-06-292008-07-01, 2008-07-072008-07-11, It was synchronized with the wide-angle infrared dual-mode line / area array scanner (widas), imaging spectrometer omis-2, TM, aster, Chris, Hyperion and other satellites.

Data processing method

Factors affecting CE318 data accuracy: local atmospheric pressure, instrument calibration parameters and various conversion factors （ 1) During the calculation of preprocessed data, the atmospheric pressure is obtained by using the empirical relationship between atmospheric pressure and elevation, which is mostly inconsistent with the reality. Therefore, in order to obtain accurate inversion results, synchronous meteorological station data is required;  （ 2) The error of instrument calibration data causes the systematic error of inversion results, so it is necessary to organize field calibration or instrument indoor calibration;  （ 3) The inversion of aerosol optical thickness and water vapor content in water vapor channel requires various conversion factors. The conversion factors are empirical parameters, and their practicability needs to be further verified. Outdoor calibration: under the condition of stable atmospheric parameters, obtain the measurement data with atmospheric mass number between 3-7, and calibrate with langly principle; Indoor calibration: standard light source.  & emsp; Factors affecting CE318 data accuracy: local atmospheric pressure, instrument calibration parameters and various conversion factors.  （ 1) During the calculation of preprocessed data, the atmospheric pressure is obtained by using the empirical relationship between atmospheric pressure and elevation, which is mostly inconsistent with the reality. Therefore, in order to obtain accurate inversion results, synchronous meteorological station data is required;  （ 2) The error of instrument calibration data causes the systematic error of inversion results, so it is necessary to organize field calibration or instrument indoor calibration;  （ 3) The inversion of aerosol optical thickness and water vapor content in water vapor channel requires various conversion factors. The conversion factors are empirical parameters, and their practicability needs to be further verified. Outdoor calibration: under the condition of stable atmospheric parameters, obtain the measurement data with atmospheric mass number between 3-7, and calibrate with langly principle; Indoor calibration: standard light source.

Data quality description

Good data quality

Base information

collect time:2008/05/20-2008/07/11

collect place:Heihe River Basin, intensive observation area of huazhaizi desert, hydrological test area of arid area in the middle reaches, intensive observation area of Yingke oasis and intensive observation area of Zhangye City

data size:4.6 MiB

data format:txt,word

Coordinate system: WGS84

Keywords

Theme:Aerosol, aerosol optical depth / thickness, infrared wide-angle dual-mode imager, widas, solar spectrophotometer, atmospheric water vapor
	Subject:physical geography 
	Time:2008
	Place:Heihe River Basin,  Intensive observation area in Zhangye City,  Infill observation in Yingke Oasis,  Hydrological experimental area in the middle reaches of arid area,  Intensive observation area of huazhaizi desert

Space scope

|  | north： 38.97 |  |
| --- | --- | --- |
| west：100.59 |  | east：100.44 |
|  | south： 38.04 |  |

Service information

contacts:Min Yufang

phone:0931-4967596

mailbox:ncdc@lzb.ac.cn

Contact information

contributors:Zhou Mengwei

author:Zhou Mengwei

manager:Zhou Mengwei

Project information

Citations and annotations

In order to protect the rights and interests of the platform's scientific and technological resources, expand the services of the platform center, and enhance the application potential of scientific and technological resources,resource users are requested to mark and quote the research results (including published papers, treatises, data products, unpublished research reports, data products, etc.) generated by the use of resources in the following manner.

For the achievements published in English, please refer to the following specifications：

The dataset is provided by National Cryosphere Desert Data Center(https://www.ncdc.ac.cn).

Data reference:

Zhou Mengwei. Joint experiment of Heihe comprehensive remote sensing: observation data set of solar spectrophotometer in dense observation area of Yingke oasis and huazhaizi desert. National Cryosphere Desert Data Center(http://www.ncdc.ac.cn), 2022. https://cstr.cn/CSTR:11738.11.ncdc.NIEER.2021.1807.

Article citation:

Wang Ying, Xue Yong, Li Yingjie, Guang Jie, Mei Linlu, Xu Hui, Ai Jianwen. Prior knowledge-supported aerosol optical depth retrieval over land surfaces at 500 m spatial resolution with MODIS data[J]. International Journal of Remote Sensing, 2012, 33, (3). 刘艳, 王锦地, 周红敏, 薛华柱. 黑河中游试验区不同分辨率LAI数据处理、分析和尺度转换[J]. 遥感技术与应用, 2010, 25, (06): 805-813. 康国婷, 阎广建, 任华忠, 王颢星, 钱永刚. 田块尺度作物辐射温度获取方法对比研究[J]. 地球科学进展, 2009, 24, (07): 784-792.