Remote sensing experiment of ecological hydrology in Heihe River Basin: Airborne plmr ground synchronous observation data set of Yingke oasis and huazhaizi desert in the middle reaches of Heihe River Basin (July 2012)

DOI：10.12072/ncdc.nieer.db3562.2023

CSTR：11738.11.ncdc.nieer.2020.1615

Share type:Apply to Access

Category:Remote Sensing

Datasets description

On July 7, 2012, airborne flight ground synchronous observation was carried out in Yingke oasis and huazhaizi desert plmr quadrat. The local surface synchronous data set can provide a basic ground data set for the development and verification of passive microwave remote sensing soil moisture retrieval algorithm. This data set includes two parts: soil moisture observation and vegetation observation. The former saves data in vector file, the spatial location is the location of each sampling point (WGS84 + UTM 47N), and the measurement information such as soil moisture is recorded in the attribute file; Vegetation sampling information is recorded in Excel table. On July 10, 2012, airborne flight ground synchronous observation was carried out in Yingke oasis and huazhaizi desert plmr quadrat. The local surface synchronous data set can provide a basic ground data set for the development and verification of passive microwave remote sensing soil moisture retrieval algorithm. This data set includes two parts: soil moisture observation and vegetation observation. The former saves data in vector file, the spatial location is the location of each sampling point (WGS84 + UTM 47N), and the measurement information such as soil moisture is recorded in the attribute file

Data source description

Plmr (polarimetric L-band multibeam radiometer) is a dual polarization (H / V) L-band microwave radiometer with center frequency of 1.413 GHz, bandwidth of 24 MHz, resolution of 1 km (relative altitude of 3 km), simultaneous observation of 6 beams, incident angle of ± 7 º, ± 21.5 º, ± 38.5 º, sensitivity & lt; 1K。 The flight mainly covers the ecological and hydrological test area of artificial oasis in the middle reaches

Data processing method

Quadrat and sampling strategy:    The observation area is located in the south edge of Zhangye Oasis anyangtan desert transition zone and the west side of Zhang (Zhangye) - Da (Daman) highway. It crosses Longqu trunk canal from north to South and is divided into two parts, 1 km in the southwest × For the 1km desert quadrat, because the desert is relatively homogeneous, the soil moisture of 5 points (1 point around and the center point. In the actual measurement process, several more points can be measured during walking along the road). The four corners are separated by 600m except the diagonal direction. The southwest corner is huazhaizi desert station, which is convenient for comparison with the data of meteorological station. On the northeast side, an area of 1.6 km was selected × Synchronous observation was carried out on the underlying surface of the oasis according to the 1.6 km large sample policy. The selection basis of quadrat is mainly to consider the representativeness of surface coverage, avoid residential houses and greenhouses as far as possible, cross oasis farmland and some deserts in the south, accessibility and observation (road consumption) time, so as to obtain the comparison with the brightness temperature observed by plmr.    Considering the resolution of plmr observation, in synchronous observation, 11 sample lines (east-west distribution) are collected at an interval of 160 m in the east-west direction, with a total of 21 points (north-south direction) at an interval of 80 m in each line. Four hydroprobe data acquisition systems (HDAS, ref. 2) are used for simultaneous measurement.    Measurement content:    About 230 points on the quadrat were obtained, and each point was observed twice, that is, for the film covered corn field, twice were observed at each sampling point, once inside the film (marked as a in the data record) and once outside the film (marked as B in the data record). Because the HDAS system adopts pogo portable soil sensor, the soil temperature, soil moisture (volume moisture content), loss tangent, soil conductivity, real and imaginary parts of soil complex dielectric are observed and obtained. Vegetation parameter observation was carried out at some representative soil water sampling points, and the plant height and biomass (vegetation water content) were measured

Data quality description

Good data quality

Base information

collect time:2012/07/07-2012/07/10

collect place:Heihe River Basin, middle reaches, artificial oasis, experimental area

data size:14.0 MiB

data format:

Coordinate system: WGS84

Keywords

Theme:Ground synchronous observation, Brightness temperature, Airborne microwave radiometer, soil moisture
	Subject:geography 
	Time:2012
	Place:Artificial oasis experimental area in the middle reaches,  Heihe River Basin

Space scope

|  | north： 38.78 |  |
| --- | --- | --- |
| west：100.33 |  | east：100.32 |
|  | south： 38.77 |  |

Service information

contacts:Min Yufang

phone:0931-4967596

mailbox:ncdc@lzb.ac.cn

Contact information

contributors:Wang shuguo

author:Wang shuguo

manager:Ma mingguo , Wang shuguo

Project information

Citations and annotations

In order to protect the rights and interests of the platform's scientific and technological resources, expand the services of the platform center, and enhance the application potential of scientific and technological resources,resource users are requested to mark and quote the research results (including published papers, treatises, data products, unpublished research reports, data products, etc.) generated by the use of resources in the following manner.

For the achievements published in English, please refer to the following specifications：

The dataset is provided by National Cryosphere Desert Data Center(https://www.ncdc.ac.cn).

Data reference:

Wang shuguo. Remote sensing experiment of ecological hydrology in Heihe River Basin: Airborne plmr ground synchronous observation data set of Yingke oasis and huazhaizi desert in the middle reaches of Heihe River Basin (July 2012). National Cryosphere Desert Data Center(http://www.ncdc.ac.cn), 2021. https://cstr.cn/CSTR:11738.11.ncdc.nieer.2020.1615.