2016-2019 runoff observation data of kalongla glacier in Southeast Tibet

DOI：10.12072/ncdc.BoMi.db0005.2020

CSTR：11738.11.ncdc.BoMi.2020.15

Share type:Apply to Access

Category:Glacier

Datasets description

This data set is the runoff data of gelongla glacier in Southeast Tibet from 2016 to 2019. The observation station is located near the garongla glacier in Bomi County of Tibet Autonomous Region, and the garongla glacier is located in the east side of the 24K garongla tunnel of Bomi County Motuo highway in Southeast Tibet, which is a typical marine glacier. The altitude of the glacier is about 902.082 km, and the area of the glacier is about 830.082 km above sea level. The upper limit of the distribution of moraine on the garongla glacier is about 4200m. The thickness of the surface Moraine in the surface moraine area is different. Both sides and ends of the glacier are thicker, and the middle area is thinner. The average thickness of the middle area is about 30cm. The observation station records the glacier runoff data in detail. The original data is obtained by the temperature and humidity sensor, tipping bucket rainfall sensor, ultrasonic water level sensor and flow velocity sensor of the Galongla glacier through the data collector. The data is collected automatically every 10 minutes, and the original data is obtained by cr800 data collector. After manual collection, the data of temperature and humidity, water depth and flow velocity are obtained According to the daily average data and sorting, rainfall data using daily cumulative data and sorting. Station Name: garongla glacier runoff observation station, longitude and latitude: 95 ° 42 ′ 23, 29 ° 49 ′ 23, altitude: 3200m, data content: humidity unit is percentage (%), temperature unit is centigrade (℃), rainfall unit is millimeter (mm), water depth unit is meter (m), flow unit is cubic meter per second (m3 / s)

Data source description

Automatic acquisition

Data processing method

The original data is automatically collected by field instruments, collected every 10 minutes, collected and summarized by professionals regularly. Daily average data is used for temperature, humidity and water depth data, and accumulated rainfall data is used for rainfall data and processed into data set products

Data quality description

Professional staff at all stages of data acquisition and processing, The quality of the data is good.

Base information

collect time:2016/07/24-2019/08/06

collect place:Bomi County, Tibet Autonomous Region

altitude:3500.0m- 4500.0m

data size:97.2 KiB

data format:

Coordinate system: WGS84

Keywords

Theme:Rainfall, glaciers, temperature, flow, humidity, water depth
	Subject:physical geography 
	Time:2016 2017 2018 2019
	Place:Garongla glacier,  Bomi County,  Tibet Autonomous Region

Space scope

|  | north： 29.98 |  |
| --- | --- | --- |
| west： 95.31 |  | east： 95.77 |
|  | south： 29.45 |  |

Service information

contacts:Li Hongxing

phone:0931-4967592

mailbox:lihongxing@lzb.ac.cn

Contact information

contributors:Chen Ningsheng,  Ding Haitao,  Deng Mingfeng

author:Ding Haitao

manager:Ding Haitao , Chen Ningsheng

Project information

Citations and annotations

In order to protect the rights and interests of the platform's scientific and technological resources, expand the services of the platform center, and enhance the application potential of scientific and technological resources,resource users are requested to mark and quote the research results (including published papers, treatises, data products, unpublished research reports, data products, etc.) generated by the use of resources in the following manner.

For the achievements published in English, please refer to the following specifications：

The dataset is provided by National Cryosphere Desert Data Center(https://www.ncdc.ac.cn).

Data reference:

Chen Ningsheng, Ding Haitao, Deng Mingfeng. 2016-2019 runoff observation data of kalongla glacier in Southeast Tibet. National Cryosphere Desert Data Center(http://www.ncdc.ac.cn), 2020. https://cstr.cn/CSTR:11738.11.ncdc.BoMi.2020.15.