Data set of land use statistics of Sanmenxia reservoir area of Yiluo River in 2013-2017

DOI：10.12072/ncdc.HHSTBC.db0030.2021

CSTR：11738.11.ncdc.HHSTBC.2020.200

Share type:Apply to Access

Category:Soil and Water Conservation

Datasets description

The data set of land use map in Sanmenxia reservoir area of Yiluo River includes the land use of Luoning County in Henan Province, Yuanqu County of Shanxi Province and Mianchi County of Henan Province in 2013, 2014, 2015, 2016 and 2017. The format of the data is xlsx. The data is named in the form of "key governance area + administrative region + year + land use statistical table", such as "×××" Statistical table of land use in key control area ×× County ×××. The classification system of land use includes 8 first class, 22 second class and 2 third class.

Data source description

The main data from USGS and Landsat were, https://glovis.usgs.gov/ ）Access, a few from China geospatial data cloud（ http://www.gscloud.cn/ ）Get

Data processing method

Based on ecognition software platform, the method of combining object-oriented computer automatic classification with manual visual interpretation was used to extract the annual land use data of the study area. Then three methods are used to verify the accuracy of the data: field survey, high-resolution image recognition and Google Earth sample point recognition. Finally, according to the classification standard of land use, the statistical table of land use is obtained.

Data quality description

The remote sensing images are preprocessed by radiation correction, orthorectification, fusion and mosaic. The actual surface area corresponding to the minimum patch area is not less than 0.1 Hm2, the polygon has no overlap, no gap, and the patch attribute has no vacancy or redundancy. Before remote sensing image interpretation, remote sensing image, typical survey and field comparison were used to establish remote sensing interpretation marks of land use. Based on the remote sensing image, combined with the interpretation marks, the land use types are extracted. Reexamination of interpretation results: take no less than 5% of the total map spots for verification. The number and results of field verification samples meet the requirements of technical specification for remote sensing monitoring of soil and water conservation (sl592-2012). For the verification map spots, 10% of the verification samples are selected for field verification.

Base information

collect time:2013/01/01-2017/12/31

collect place:

data format:

Coordinate system: CGCS2000

Projection

Keywords

Theme:According to the statistical table of land use, there are 8 primary elements
	Subject:
	Time:2013 2014 2015 2016 2017
	Place:Yuanqu County,  Luoning County,  Sanmenxia reservoir area of Yiluo River,  Shanxi Province,  Henan Province,  Mianchi County

Space scope

|  | north： 35.42 |  |
| --- | --- | --- |
| west：111.13 |  | east：112.08 |
|  | south： 34.09 |  |

Service information

contacts:

phone:0931-4967596

mailbox:ncdc@lzb.ac.cn

Contact information

contributors:Monitoring Center for ecological environment of soil and water conservation in the Yellow River Basin

author:Li Suya

manager:Li Suya

Project information

Citations and annotations

In order to protect the rights and interests of the platform's scientific and technological resources, expand the services of the platform center, and enhance the application potential of scientific and technological resources,resource users are requested to mark and quote the research results (including published papers, treatises, data products, unpublished research reports, data products, etc.) generated by the use of resources in the following manner.

For the achievements published in English, please refer to the following specifications：

The dataset is provided by National Cryosphere Desert Data Center(https://www.ncdc.ac.cn).

Data reference:

Monitoring Center for ecological environment of soil and water conservation in the Yellow River Basin. Data set of land use statistics of Sanmenxia reservoir area of Yiluo River in 2013-2017. National Cryosphere Desert Data Center(http://www.ncdc.ac.cn), 2021. https://cstr.cn/CSTR:11738.11.ncdc.HHSTBC.2020.200.