Observation data set of runoff and sediment in wangmaogou watershed in the first sub region of Loess Hilly and gully region from 1991 to 2000

DOI：10.12072/ncdc.HWSDZhan.db0054.2020

CSTR：11738.11.ncdc.HWSDZhan.2020.118

Share type:Apply to Access

Category:Soil and Water Conservation

Datasets description

This data set is the runoff and sediment observation data of a runoff station in wangmaogou watershed from 1991 to 2000. Through the water level observation and sediment sampling of the runoff station, the runoff and sediment yield are calculated. The data set includes flow section diagram of runoff station, measured discharge result table, extraction table of flood hydrological elements, successive flood test result table, daily average discharge table, daily average sediment concentration table and daily average sediment transport rate table. See the basic information table of runoff stations in wangmaogou basin for the situation of runoff stations.

Data source description

Manual timing observation

Data processing method

1. The water level observation method of runoff station is: read the water gauge number of the flow measurement building on the flow measurement section. In non flood season, 8:00 and 20:00 respectively are observed once a day. If there is rainfall, observe twice at 8:00 and 20:00 respectively when the daily water level changes slowly; observe four times at 2:00, 8:00, 14:00 and 20:00 respectively when the daily water level changes greatly or there are slow peaks and valleys. During the flood season, when the water level changes violently, it shall be observed in multiple of 6 minutes. 2. Flow measurement method of wangmaozhuang runoff station: adopt the calibrated natural section observation and the calibrated push flow formula for flow calculation. 3. Sediment sampling and treatment: the sediment in the flow measurement section is sampled by the thalweg buoy method, and the number of sediment sampling depends on the number of changes in the flood water level. In order to control the change of sediment concentration, the number of sampling is increased during the flood peak. The sample was treated by displacement method.

Data quality description

Manual observation shall be conducted by professional technicians; during data entry, four levels of manual verification shall be conducted, including tabulation, primary calibration, re calibration and audit, so as to ensure data quality.

Base information

collect time:1991/01/01-2000/12/31

collect place:

altitude:2.0m- Nonem

data size:370.8 KiB

data format:

Coordinate system: WGS84

Keywords

Theme:Runoff, Sediment concentration, Sediment transport rate, Runoff coefficient, Runoff modulus, Soil loss, water level, Erosion modulus, Runoff depth, flow
	Subject:Hydrometeorology 
	Time:1991 1992 1993 1994 1995 1996 1997 1998 1999 2000
	Place:Yulin,  Suide,  Wang Maogou,  Shaanxi Province

Space scope

|  | north： 37.59 |  |
| --- | --- | --- |
| west：110.34 |  | east：110.38 |
|  | south： 37.57 |  |

Service information

contacts:Min Yu Fang

phone:093149675976

mailbox:myf@lzb.ac.cn

Contact information

contributors:Suide administration and Supervision Bureau of water and soil conservation of the Yellow River (Suide scientific experimental station of water and soil conservation)

author:LEI Xin ,  LiuSijun

manager:GUO Yumei , Suide administration and Supervision Bureau of water and soil conservation of the Yellow River (Suide scientific experimental station of water and soil conservation)

Project information

Citations and annotations

In order to protect the rights and interests of the platform's scientific and technological resources, expand the services of the platform center, and enhance the application potential of scientific and technological resources,resource users are requested to mark and quote the research results (including published papers, treatises, data products, unpublished research reports, data products, etc.) generated by the use of resources in the following manner.

For the achievements published in English, please refer to the following specifications：

The dataset is provided by National Cryosphere Desert Data Center(https://www.ncdc.ac.cn).

Data reference:

Suide administration and Supervision Bureau of water and soil conservation of the Yellow River (Suide scientific experimental station of water and soil conservation). Observation data set of runoff and sediment in wangmaogou watershed in the first sub region of Loess Hilly and gully region from 1991 to 2000. National Cryosphere Desert Data Center(http://www.ncdc.ac.cn), 2020. https://cstr.cn/CSTR:11738.11.ncdc.HWSDZhan.2020.118.