Data set of fracture types of sandstone sound sources under the action of multiple factors in freeze-thaw environments (2024.7)

DOI：10.12072/ncdc.db7685.2026

CSTR：11738.11.ncdc.db7685.2026

Share type:Apply to Access

Category:Others

Datasets description

Data source description

1. Research object: Slope sandstone samples in alpine areas   2. Research background: Acoustic signal collection during sample loading can accurately determine the source of rupture and the type of rupture   3. Test instrument: Acoustic emission acquisition system, American Physical Acoustics Corporation (PAC) Micro-II Express (8 channels)   Parameters: Sensor: 55 kHz resonant frequency (35 - 100 kHz bandwidth) Sampling rate: 10 MSPs Threshold: 40 dB   4. Data content and type   Raw data: RA value, AF value, spatio-temporal coordinates of sound source   Processed data: RA-AF data density distribution map, crack type proportion classification results, three-dimensional sound source positioning data set   Data format: word, image (png/jpg)

Data processing method

1. Raw data source: Collect raw waveform parameters from the acoustic emission monitoring system   2. Preprocessing steps   Denoising: filtering, thresholding, background noise correction   Standardization: Unify Timeline and unit conversion   Data cleaning: Remove invalid waveforms and instrument interference signals   3. Feature parameter extraction   Calculate time-domain parameters from the waveform: rise time, amplitude, duration, ringing calculation, number of events   Calculate RA value, AF value, and sound source localization point according to formula   4. Data analysis   Draw RA-AF data density map and complete the identification of rupture crack types   Multi-channel arrival time difference algorithm is used to locate three-dimensional sound source, and the spatio-temporal distribution of crack source points is obtained   5. Storage and organization   The processing results are saved in word, png/jpg format

Data quality description

1. The parameter setting range covers the actual working conditions in high-altitude mining areas

Base information

collect time:2024/07/01-2024/07/31

collect place:Muli Coalfield Pit No. 4

data size:23.2 MiB

data format:word，JPG

Coordinate system: 无

Keywords

Theme:Keywords alpine Kuangqu, freeze-thaw action, sandstone slope, acoustic emission, RA-AF value, rupture mode
	Subject:geology 
	Time:2024
	Place:Pit 4 in Muli Coalfield,  Qinghai Province

Space scope

|  | north： |  |
| --- | --- | --- |
| west： |  | east： |
|  | south： |  |

Service information

contacts:None

phone:13259922729

mailbox:xupy@xust.edu.cn

Contact information

contributors:xu pei yun,  zhang xiao long

author:zhang xiao long

manager:xu pei yun

Project information

number:2022YFF1302601，
      name:Integration of Technologies for Monitoring Spatiotemporal Changes in the Hydrological Ecology of Permafrost in High-Altitude Mining Areas and Data Fusion

Citations and annotations

In order to protect the rights and interests of the platform's scientific and technological resources, expand the services of the platform center, and enhance the application potential of scientific and technological resources,resource users are requested to mark and quote the research results (including published papers, treatises, data products, unpublished research reports, data products, etc.) generated by the use of resources in the following manner.

For the achievements published in English, please refer to the following specifications：

The dataset is provided by National Cryosphere Desert Data Center(https://www.ncdc.ac.cn).

Data reference:

xu pei yun, zhang xiao long. Data set of fracture types of sandstone sound sources under the action of multiple factors in freeze-thaw environments (2024.7). National Cryosphere Desert Data Center(http://www.ncdc.ac.cn), 2026. https://cstr.cn/CSTR:11738.11.ncdc.db7685.2026.