Typical soil hydrothermal and mechanical parameter data of the permafrost region in Northeast China (2023)

DOI：10.12072/ncdc.nieer.db7287.2026

CSTR：11738.11.NCDC.NIEER.DB7287.2026

Share type:Apply to Access

Category:Frozen soil

Datasets description

Data source description

The dataset is based on collected soil samples, and the laboratory conducted single measurements for each indicator. Simultaneously measure three samples for each parameter and take their mean to reflect the typical hydraulic and thermal characteristics of the soil.

Data processing method

Based on soil samples collected along the China Russia crude oil pipeline, raw hydrothermal data were generated through laboratory testing and analysis. Each indicator is measured simultaneously for three samples, and the mean is taken to form the final dataset.     The laboratory measures the water content, density, specific gravity, organic matter content, liquid limit, plastic limit, thaw settlement coefficient, compression coefficient, freezing temperature, thermal conductivity, volumetric heat capacity and other hydrothermal indicators for each sample. The testing follows the "Code for Soil Testing" (GB/T 50123-2019) and relevant national standards and specifications, and standard laboratory instruments are used for measurement. Simultaneously measure three samples for each indicator and take their mean to form the final dataset, in order to reduce experimental errors and ensure data reliability.

Data quality description

This dataset is based on indoor geotechnical tests conducted on soil samples from typical sites along the China Russia crude oil pipeline. The original observation data was processed through sorting, parameter calculation, unit unification, and quality control. The experimental and calculation methods strictly follow the GB/T 50123-2019 "Soil Test Method Standard".     The data accuracy is consistent with the indoor test measurement accuracy, and the data generation and processing process is complete, standardized, and traceable. Quality control measures such as integrity and rationality checks are taken to ensure data quality.     The data is applicable for the analysis of freeze-thaw process, simulation of water heat force coupling, and risk assessment of freeze-thaw disasters under the conditions of buried warm oil pipeline engineering in permafrost regions.

Base information

collect time:2023/04/01-2023/04/30

collect place:Northeast Permafrost Region

data size:128.0 KiB

data format:*.xlsx

Coordinate system: 无

Keywords

Theme:Permafrost, hydrothermal parameters, China Russia crude oil pipeline
	Subject:geography 
	Time:2023
	Place:Northeast Permafrost Region

Space scope

|  | north： 53.33 |  |
| --- | --- | --- |
| west：124.01 |  | east：124.70 |
|  | south： 51.31 |  |

Service information

contacts:

phone:0931-4967596

mailbox:ncdc@lzb.ac.cn

Contact information

contributors:Li Guoyu,  Chen Dun,  Gao Kai,  Wang Fei,  Cao Yapeng,  Yin Guoan,  NIU Fujun,  Shang Yunhu,  Lin Juncen,  Du Qingsong,  Jia Han,  Wang Yapeng,  Qi Shunshun,  Ai Xihui,  Yang Jiawei,  Li Zhaoxiang,  Hu Meirong

author:Gao Kai ,  Chen Dun

manager:Chen Dun

Project information

number:2022FY100700，
      name:Survey of Permafrost Conditions and Freeze-Thaw Damage in the High-Latitude Regions of Northeast China

Citations and annotations

In order to protect the rights and interests of the platform's scientific and technological resources, expand the services of the platform center, and enhance the application potential of scientific and technological resources,resource users are requested to mark and quote the research results (including published papers, treatises, data products, unpublished research reports, data products, etc.) generated by the use of resources in the following manner.

For the achievements published in English, please refer to the following specifications：

The dataset is provided by National Cryosphere Desert Data Center(https://www.ncdc.ac.cn).

Data reference:

Li Guoyu, Chen Dun, Gao Kai, Wang Fei, Cao Yapeng, Yin Guoan, NIU Fujun, Shang Yunhu, Lin Juncen, Du Qingsong, Jia Han, Wang Yapeng, Qi Shunshun, Ai Xihui, Yang Jiawei, Li Zhaoxiang, Hu Meirong. Typical soil hydrothermal and mechanical parameter data of the permafrost region in Northeast China (2023). National Cryosphere Desert Data Center(http://www.ncdc.ac.cn), 2026. https://cstr.cn/CSTR:11738.11.NCDC.NIEER.DB7287.2026.