2012 Dongkemadi GPS control points joint survey data set

DOI：10.12072/ncdc.DIMaC.db0064.2020

CSTR：11738.11.ncdc.DIMaC.2020.97

Share type:Apply to Access

Category:Glacier

Datasets description

This data is the joint survey data of control points from hezhongsizhuang E650 static measurement in June 2012, and the result solution accuracy is controlled within 2cm through three national control points; the data format is excel file format, and the record content includes four fields of point name, longitude, latitude and elevation, including six point measurement records, which is publicly shared. The data set contains the field GPS control point joint survey results in Dongkemadi area in 2012, and the file is excel data file. In the data file, longitude, unit degree (°), accurate to 8 decimal places; latitude, unit degree (°), accurate to 8 decimal places; elevation, that is, the glacier surface elevation of the current year, unit meter (m), accurate to 2 decimal places.

Data source description

Self generated.

Data processing method

This data is the joint survey data of control points from hezhongsizhuang E650 static measurement in June 2012. Through the adjustment of three national control points with landtop2.05 tool, the result solution accuracy is controlled within 2cm, and the data longitude and latitude coordinate system UTM WGS84. Among them, the GPS E650 static measurement was carried out according to the "GPS survey specification" (GB / t18314-2001) and the data from the field survey of "glacier observation and research methods" (Science Press, 2012).

Data quality description

4-6 E650 are used for joint survey, the synchronous measurement time is more than 1 hour, and 1-3 national control points are used for joint survey; the result calculation accuracy is controlled within 2cm

Base information

collect time:2012/06/01-2012/06/30

collect place:Dongkemadi

data size:30.4 KiB

data format:

Coordinate system: WGS84

Keywords

Theme:GPS, Dongkemadi, control point
	Subject:Geometric geodesy 
	Time:2012
	Place:Dongkemadi

Space scope

|  | north： 33.33 |  |
| --- | --- | --- |
| west： 91.74 |  | east： 92.27 |
|  | south： 32.78 |  |

Service information

contacts:Li Hongxing

phone:0931-4967592

mailbox:lihongxing@lzb.ac.cn

Contact information

contributors:Shangguan donghui

author:WU Kunpeng

manager:Shangguan donghui

Project information

number:2013FY111400，
      name:

Citations and annotations

In order to protect the rights and interests of the platform's scientific and technological resources, expand the services of the platform center, and enhance the application potential of scientific and technological resources,resource users are requested to mark and quote the research results (including published papers, treatises, data products, unpublished research reports, data products, etc.) generated by the use of resources in the following manner.

For the achievements published in English, please refer to the following specifications：

The dataset is provided by National Cryosphere Desert Data Center(https://www.ncdc.ac.cn).

Data reference:

Shangguan donghui. 2012 Dongkemadi GPS control points joint survey data set. National Cryosphere Desert Data Center(http://www.ncdc.ac.cn), 2020. https://cstr.cn/CSTR:11738.11.ncdc.DIMaC.2020.97.