Comprehensive permafrost ground temperature dataset along the Jiamo Highway in Northeast China (2019-2023)

DOI：10.12072/ncdc.nieer.db7292.2026

CSTR：11738.11.NCDC.NIEER.DB7292.2026

Share type:Apply to Access

Category:Frozen soil

Datasets description

Data source description

The original observation data was automatically collected through the thermistor temperature chain (developed by the State Key Laboratory of Cryosphere Science and Frozen Soil Engineering, with a measurement accuracy of ± 0.05 ℃) deployed on the site, and recorded by the CR3000 data acquisition instrument and the TRM128 deep hole temperature measurement expansion board, with a collection frequency of 2h/time.

Data processing method

Set up 4 boreholes on site and use thermistor temperature chains for automatic observation, with a measurement accuracy of ± 0.05 ℃; The acquisition unit includes the CR3000 data acquisition instrument from Campbell Scientific in the United States and the TRM128 deep hole temperature measurement expansion board. The temperature probe is installed at a depth covering 0 to 25 meters, with a collection frequency of once every 2 hours.     Following standard specifications: The observation method complies with field monitoring standards and relevant technical requirements to ensure the accuracy and representativeness of monitoring data.     Processing method: The raw observation data is calculated on a daily basis to generate a daily average ground temperature dataset; The data has not been interpolated, simulated, or subjected to any other secondary calculation processing.

Data quality description

The collection equipment is regularly calibrated to ensure the continuity, accuracy, and scientific applicability of the data.

Base information

collect time:2019/08/01-2023/06/03

collect place:Northeast Permafrost Region

data size:799.2 KiB

data format:*.xlsx

Coordinate system: 无

Keywords

Theme:Observation of frozen soil and ground temperature along the Jiamo Highway, Permafrost, Ground Temperature Monitoring
	Subject:geography 
	Time:2019 2020 2021 2022 2023
	Place:Northeast Permafrost Region

Space scope

|  | north： 51.24 |  |
| --- | --- | --- |
| west：124.19 |  | east：124.31 |
|  | south： 50.47 |  |

Service information

contacts:

phone:0931-4967596

mailbox:ncdc@lzb.ac.cn

Contact information

contributors:Li Guoyu,  Chen Dun,  Gao Kai,  Wang Fei,  Cao Yapeng,  Yin Guoan,  NIU Fujun,  Shang Yunhu,  Lin Juncen,  Du Qingsong,  Jia Han,  Wang Yapeng,  Qi Shunshun,  Ai Xihui,  Yang Jiawei,  Li Zhaoxiang,  Hu Meirong

author:Gao Kai ,  Chen Dun

manager:Chen Dun

Project information

number:2022FY100700，
      name:Survey of Permafrost Conditions and Freeze-Thaw Damage in the High-Latitude Regions of Northeast China

Citations and annotations

In order to protect the rights and interests of the platform's scientific and technological resources, expand the services of the platform center, and enhance the application potential of scientific and technological resources,resource users are requested to mark and quote the research results (including published papers, treatises, data products, unpublished research reports, data products, etc.) generated by the use of resources in the following manner.

For the achievements published in English, please refer to the following specifications：

The dataset is provided by National Cryosphere Desert Data Center(https://www.ncdc.ac.cn).

Data reference:

Li Guoyu, Chen Dun, Gao Kai, Wang Fei, Cao Yapeng, Yin Guoan, NIU Fujun, Shang Yunhu, Lin Juncen, Du Qingsong, Jia Han, Wang Yapeng, Qi Shunshun, Ai Xihui, Yang Jiawei, Li Zhaoxiang, Hu Meirong. Comprehensive permafrost ground temperature dataset along the Jiamo Highway in Northeast China (2019-2023). National Cryosphere Desert Data Center(http://www.ncdc.ac.cn), 2026. https://cstr.cn/CSTR:11738.11.NCDC.NIEER.DB7292.2026.