Data set of land use statistics of national key prevention areas of Tarim River Oasis in 2015-2017

DOI：10.12072/ncdc.HHSTBC.db0038.2021

CSTR：11738.11.ncdc.HHSTBC.2020.202

Share type:Apply to Access

Category:Soil and Water Conservation

Datasets description

Data source description

The data source is Landsat TM and ETM + data, mainly obtained from USGS (https://glovis.usgs.gov/), and a few from China geospatial data cloud (http://www.gscloud.cn/)

Data processing method

Based on ecognition software platform, the method of object-oriented computer automatic classification and manual visual interpretation is used to extract the land use data of the study area year by year. Then three methods are used to verify the data accuracy: field sample point survey, high-resolution image recognition and Google Earth sample point recognition. Finally, according to the classification standard of land use, the statistical table of land use is obtained.

Data quality description

Remote-sensing images are pre-processed by radiation correction, orthorectification, and fusion and mosaic. The actual surface area corresponding to the minimum map area is not less than 0.1hm2, the polygons have no overlap, no gaps, and the map properties are not vacant or redundant. Before interpreting remote sensing images, the remote sensing imagery, typical surveys, and field comparison methods are used to establish the land use remote sensing interpretation marks. Based on the remote sensing image, combined with the interpretation mark, the land use type is extracted. Interpretation of interpretation results: extract no less than 5% of the total patch for verification. The number and results of field verification samples meet the requirements of the Technical Specifications for Remote Sensing Monitoring of Soil and Water Conservation (SL592-2012). For verification maps, 10% are selected as verification samples for field verification.

Base information

collect time:2015/01/01-2017/12/31

collect place:Tarim River National key prevention area

data size:34.7 KiB

data format:

Coordinate system: 无

Keywords

Theme:There are 8 first-class elements
	Subject:
	Time:2015 2016 2017
	Place:Tarim River Oasis,  Yuli County,  Xinjiang Uygur Autonomous Region,  Ruoqiang County

Space scope

|  | north： 41.66 |  |
| --- | --- | --- |
| west： 84.02 |  | east： 93.77 |
|  | south： 36.02 |  |

Service information

contacts:

phone:0931-4967596

mailbox:ncdc@lzb.ac.cn

Contact information

contributors:Monitoring Center for ecological environment of soil and water conservation in the Yellow River Basin

author:Li Suya

manager:Li Suya

Project information

Citations and annotations

In order to protect the rights and interests of the platform's scientific and technological resources, expand the services of the platform center, and enhance the application potential of scientific and technological resources,resource users are requested to mark and quote the research results (including published papers, treatises, data products, unpublished research reports, data products, etc.) generated by the use of resources in the following manner.

For the achievements published in English, please refer to the following specifications：

The dataset is provided by National Cryosphere Desert Data Center(https://www.ncdc.ac.cn).

Data reference:

Monitoring Center for ecological environment of soil and water conservation in the Yellow River Basin. Data set of land use statistics of national key prevention areas of Tarim River Oasis in 2015-2017. National Cryosphere Desert Data Center(http://www.ncdc.ac.cn), 2021. https://cstr.cn/CSTR:11738.11.ncdc.HHSTBC.2020.202.