Digital elevation model dataset of the main peak area of Nyingchi Tanggula in 2014

DOI：10.12072/ncdc.DIMaC.db0053.2020

CSTR：11738.11.ncdc.DIMaC.2020.114

Share type:Apply to Access

Category:Glacier

Datasets description

This data set is aimed at the main peak area of Nyainqentanglha mountain, including glacier and surrounding area in the main peak area of Nyainqentanglha mountain, and the digital elevation model of this area in 2014 obtained by using remote sensing images. The data set contains one band, and the complex data of the same ground object can be obtained by using the platform equipped with synthetic aperture radar sensor at the same time or two parallel observations. After accurate registration, complex interferograms can be obtained by conjugate multiplication of complex images. Based on the geometric relationship among flight platform, beam view and baseline, the digital elevation model of glacier area is obtained by unwrapping the phase information of complex interferogram. It is only aimed at the glacier surface elevation in 2014, so it is not necessary to supplement and improve it, and fully open sharing is adopted. The data set contains dem2014, a total of 1 band. Data set file name and band name are as follows: data set file name: 2014 Nianqing Tanggula Mountain main peak area digital elevation model data set, dem2014-2014 area digital elevation model, unit: meter (m).

Data source description

This data set is mainly interpreted data, and the main data source is TerraSAR-X / TanDEM-X microwave remote sensing data.

Data processing method

The complex data of the same object on the ground can be obtained by using the platform equipped with synthetic aperture radar (SAR) sensor and observing at the same time or twice in parallel. After accurate registration, complex interferograms can be obtained by conjugate multiplication of complex images. Based on the geometric relationship among flight platform, beam view and baseline, the digital elevation model of glacier area is obtained by unwrapping the phase information of complex interferogram.

Data quality description

By using the platform equipped with TerraSAR-X and TanDEM-X sensors, the complex data (including amplitude and phase information) of the same ground object are obtained at the same time to form an interference pair. After accurate registration, complex interferograms can be obtained by conjugate multiplication of complex images. Based on the geometric relationship among flight platform, beam view and baseline, the phase information in complex interferogram is unwrapped to obtain the ground elevation information. The absolute accuracy in the vertical direction is better than 10 m, and the relative accuracy is 4 m at & gt; 20% and 2 m at & lt; 20%.

Base information

collect time:2014/03/01-2014/03/31

collect place:Main peak of Nianqing Tanggula Mountain

data size:244.0 KiB

data format:

Coordinate system: WGS84

Keywords

Theme:The main peak of Nianqing Tanggula Mountain, DEM
	Subject:
	Time:2014
	Place:The main peak of Nianqing Tanggula Mountain

Space scope

|  | north： 30.79 |  |
| --- | --- | --- |
| west： 89.96 |  | east： 91.03 |
|  | south： 29.86 |  |

Service information

contacts:Li Hongxing

phone:0931-4967592

mailbox:lihongxing@lzb.ac.cn

Contact information

contributors:LIU Shiyin,  WU Kunpeng

author:WU Kunpeng

manager:WU Kunpeng , LIU Shiyin

Project information

number:2013FY111400，
      name:

Citations and annotations

In order to protect the rights and interests of the platform's scientific and technological resources, expand the services of the platform center, and enhance the application potential of scientific and technological resources,resource users are requested to mark and quote the research results (including published papers, treatises, data products, unpublished research reports, data products, etc.) generated by the use of resources in the following manner.

For the achievements published in English, please refer to the following specifications：

The dataset is provided by National Cryosphere Desert Data Center(https://www.ncdc.ac.cn).

Data reference:

LIU Shiyin, WU Kunpeng. Digital elevation model dataset of the main peak area of Nyingchi Tanggula in 2014. National Cryosphere Desert Data Center(http://www.ncdc.ac.cn), 2020. https://cstr.cn/CSTR:11738.11.ncdc.DIMaC.2020.114.