Velocity of glacier movement at Urumqi River Source No. 1, 1985-1999

DOI：10.12072/ncdc.tianshan.db2906.2023

CSTR：11738.11.ncdc.Tianshan.2020.61

Share type:Login to Access

Category:Glacier

Datasets description

This dataset is the glacier movement velocity of glacier No.1 in Urumqi River source, Tianshan Mountain, 1985-1999.The surface movement velocity of glacier No.1 is measured by precision optical latitude and longitude instrument and total station, the former, from the control point on the glacier to the flower pole laid inside the ice to repeat the forward rendezvous, to calculate by the coordinate method, and to find out the spatial displacement of the flower pole (the body of the ice) in a unit of time and obtain it. Observations are made once a year in May at the beginning of the ablation and once a year at the end of August at the end of the ablation, so that flow velocity information can be obtained for the ablation period and for the whole year. In the latter case, the observation of the speed of movement on the glacier surface is measured using a total station, and the spatial displacement of the ice body per unit of time is calculated by the coordinate method from the control points K1 and K2 on the measuring rods laid on the glacier surface. This dataset is primarily a record of glacier movement velocities (summer, winter and annual velocities).

Data source description

Velocity of glacier movement, the distance traveled by a glacier per unit of time. Because glaciers move very slowly, the unit is often measured in centimeters per day and night or kilometers per year. It is calculated by field measurements or by some empirical formulas.

Data processing method

The spatial displacement of the ice body per unit time is calculated by the coordinate method.

Data quality description

Data quality is good.

Base information

collect time:1988/06/04-1996/09/05

collect place:Tianshan Glacier Station

altitude:3700.0m- 4200.0m

data size:133.0 KiB

data format:csv

Coordinate system: WGS84

Keywords

Theme:Movement Speed, Melting, Accumulation, Glacier Change
	Subject:physical geography 
	Time:1985 1986 1987 1988 1989 1990 1991 1992 1993 1994 1995 1996 1997 1998 1999
	Place:XinJiang,  TianShanStation

Space scope

|  | north： 43.12 |  |
| --- | --- | --- |
| west： 86.81 |  | east： 86.81 |
|  | south： 43.11 |  |

Service information

contacts:Wang Feiteng

phone:0931-4967383

mailbox:wangfeiteng@lzb.ac.cn

Contact information

contributors:LI Zhongqin,  WANG Feiteng

author:WANG Feiteng

manager:LI Zhongqin

Project information

Citations and annotations

In order to protect the rights and interests of the platform's scientific and technological resources, expand the services of the platform center, and enhance the application potential of scientific and technological resources,resource users are requested to mark and quote the research results (including published papers, treatises, data products, unpublished research reports, data products, etc.) generated by the use of resources in the following manner.

For the achievements published in English, please refer to the following specifications：

The dataset is provided by National Cryosphere Desert Data Center(https://www.ncdc.ac.cn).

Data reference:

LI Zhongqin, WANG Feiteng . Velocity of glacier movement at Urumqi River Source No. 1, 1985-1999. National Cryosphere Desert Data Center(http://www.ncdc.ac.cn), 2020. https://cstr.cn/CSTR:11738.11.ncdc.Tianshan.2020.61.