Heihe River eco hydrological remote sensing experiment: synchronous observation data set of surface temperature in the lower reaches of Heihe River (August 1, 2014)

DOI：10.12072/ncdc.nieer.db3490.2023

CSTR：11738.11.ncdc.NIEER.2021.33

Share type:Apply to Access

Category:Remote Sensing

Datasets description

The purpose of the synchronous observation of the surface temperature in the downstream eco hydrological experimental area is to obtain the synchronous surface temperature of the underlying surface such as high coverage uniform vegetation, water body and cement ground during the flight of the thermal imager, so as to support the verification and scale effect analysis of the inversion of the surface temperature from the flight data of the aerial thermal imager, and realize the authenticity test of the remote sensing products of the surface temperature. 1. Observation time August 1, 2014 2. Observation content Three large-area and uniform underlying surfaces of bitter beans, cement land and water body in the flight area were selected for synchronous observation 3. Observation mode When the thermal imager flies over the corresponding underlying surface, the hand-held infrared thermometer is used for continuous manual synchronous observation 4. Parameters and calibration of observation instruments The specific emissivity of the hand-held infrared thermometer used for observation is set to 0.95, and the field angle of view of the hand-held infrared thermometer is 1 °. At the same time, the observation instruments were calibrated on July 31, 2014. 5. Data storage All observation data are stored in Excel format

Data source description

The purpose of the synchronous observation of the surface temperature in the downstream eco hydrological experimental area is to obtain the synchronous surface temperature of the underlying surface such as high coverage uniform vegetation, water body and cement ground during the flight of the thermal imager, so as to support the verification and scale effect analysis of the inversion of the surface temperature from the flight data of the aerial thermal imager, and realize the authenticity test of the remote sensing products of the surface temperature

Data processing method

1. Observation time    August 1, 2014    2. Observation content    Three large-area and uniform underlying surfaces of bitter beans, cement land and water body in the flight area were selected for synchronous observation    3. Observation mode    When the thermal imager flies over the corresponding underlying surface, the hand-held infrared thermometer is used for continuous manual synchronous observation   4. Parameters and calibration of observation instruments    The specific emissivity of the hand-held infrared thermometer used for observation is set to 0.95, and the field angle of view of the hand-held infrared thermometer is 1 °. At the same time, the observation instruments were calibrated on July 31, 2014.    5. Data storage    All observation data are stored in Excel format

Data quality description

Good data quality

Base information

collect time:2014/08/01-2014/08/01

collect place:Heihe River Basin, downstream natural oasis, experimental area

data size:9.7 MiB

data format:

Coordinate system: WGS84

Keywords

Theme:Soil, thermal imager, soil temperature, airborne and ground remote sensing
	Subject:physical geography 
	Time:2014
	Place:Heihe River Basin,  downstream natural oasis

Space scope

|  | north： 42.03 |  |
| --- | --- | --- |
| west：101.03 |  | east：101.53 |
|  | south： 41.95 |  |

Service information

contacts:Min Yufang

phone: 0931-4967596

mailbox:ncdc@lzb.ac.cn

Contact information

contributors:

author:Ma mingguo

manager:Ma mingguo

Project information

number:91125004，
      name:

Citations and annotations

In order to protect the rights and interests of the platform's scientific and technological resources, expand the services of the platform center, and enhance the application potential of scientific and technological resources,resource users are requested to mark and quote the research results (including published papers, treatises, data products, unpublished research reports, data products, etc.) generated by the use of resources in the following manner.

For the achievements published in English, please refer to the following specifications：

The dataset is provided by National Cryosphere Desert Data Center(https://www.ncdc.ac.cn).

Data reference:

. Heihe River eco hydrological remote sensing experiment: synchronous observation data set of surface temperature in the lower reaches of Heihe River (August 1, 2014). National Cryosphere Desert Data Center(http://www.ncdc.ac.cn), 2021. https://cstr.cn/CSTR:11738.11.ncdc.NIEER.2021.33.