Comprehensive permafrost ground temperature dataset along the China-Russia Crude Oil Pipeline in Northeast China (2022-2024)

DOI：10.12072/ncdc.nieer.db7290.2026

CSTR：11738.11.NCDC.NIEER.DB7290.2026

Share type:Apply to Access

Category:Frozen soil

Datasets description

Data source description

This dataset belongs to first-hand observation data and was generated independently. The data is obtained through long-term field monitoring and electronically organized to form a standardized dataset.

Data processing method

Six monitoring sites were set up on site, with a total of eight boreholes. Thermistor temperature chains were used for automatic observation, with a measurement accuracy of ± 0.05 ℃; The acquisition unit includes the CR3000 data acquisition instrument from Campbell Scientific in the United States and the TRM128 deep hole temperature measurement expansion board. The temperature probe is installed at a depth covering 0 to 25 meters, with a collection frequency of once every 2 hours. Following standard specifications: The observation method complies with field monitoring standards and relevant technical requirements to ensure the accuracy and representativeness of monitoring data. Processing method: The raw observation data is calculated on a daily basis to generate a daily average ground temperature dataset; The data has not been interpolated, simulated, or subjected to any other secondary calculation processing. Error control: The collection equipment is regularly calibrated to ensure the continuity, accuracy, and scientific applicability of the data.

Data quality description

This data set is the first-hand observation data on the site, which is generated independently. It uses the thermistor temperature chain developed by the State Key Laboratory of Cryosphere Science and Frozen Soil Engineering for automatic collection, and the collection frequency is 2h/time. The original observation data is calculated on a daily basis to generate daily scale data, which maintains the original temperature change characteristics, ensures the accuracy, continuity, and availability of the data, and is suitable for analyzing the thermal characteristics of frozen soil, studying the stability of frozen soil around pipelines, and scientific research on frozen soil environment.

Base information

collect time:2022/10/01-2024/10/31

collect place:Northeast Permafrost Region

data size:2.0 MiB

data format:*.xlsx

Coordinate system: 无

Keywords

Theme:China Russia crude oil pipeline, frozen soil, ground temperature observation
	Subject:geography 
	Time:2022 2023 2024
	Place:Northeast Permafrost Region

Space scope

|  | north： 52.75 |  |
| --- | --- | --- |
| west：124.19 |  | east：124.70 |
|  | south： 50.47 |  |

Service information

contacts:

phone:0931-4967596

mailbox:ncdc@lzb.ac.cn

Contact information

contributors:Li Guoyu,  Chen Dun,  Gao Kai,  Wang Fei,  Cao Yapeng,  Yin Guoan,  NIU Fujun,  Shang Yunhu,  Lin Juncen,  Du Qingsong,  Jia Han,  Wang Yapeng,  Qi Shunshun,  Ai Xihui,  Yang Jiawei,  Li Zhaoxiang,  Hu Meirong

author:Gao Kai ,  Chen Dun

manager:Chen Dun

Project information

number:2022FY100700，
      name:Survey of Permafrost Conditions and Freeze-Thaw Damage in the High-Latitude Regions of Northeast China

Citations and annotations

In order to protect the rights and interests of the platform's scientific and technological resources, expand the services of the platform center, and enhance the application potential of scientific and technological resources,resource users are requested to mark and quote the research results (including published papers, treatises, data products, unpublished research reports, data products, etc.) generated by the use of resources in the following manner.

For the achievements published in English, please refer to the following specifications：

The dataset is provided by National Cryosphere Desert Data Center(https://www.ncdc.ac.cn).

Data reference:

Li Guoyu, Chen Dun, Gao Kai, Wang Fei, Cao Yapeng, Yin Guoan, NIU Fujun, Shang Yunhu, Lin Juncen, Du Qingsong, Jia Han, Wang Yapeng, Qi Shunshun, Ai Xihui, Yang Jiawei, Li Zhaoxiang, Hu Meirong. Comprehensive permafrost ground temperature dataset along the China-Russia Crude Oil Pipeline in Northeast China (2022-2024). National Cryosphere Desert Data Center(http://www.ncdc.ac.cn), 2026. https://cstr.cn/CSTR:11738.11.NCDC.NIEER.DB7290.2026.