2014 Bogda GPS survey data set

DOI：10.12072/ncdc.DIMaC.db0063.2020

CSTR：11738.11.ncdc.DIMaC.2020.93

Share type:Apply to Access

Category:Glacier

Datasets description

This data is the joint survey data of control points by using Huace X20 static measurement in June 2014. There is no national control point, and the synchronous measurement time is more than 1 hour. The result calculation accuracy is controlled within 2cm. The data format is excel file format. The record content includes 4 fields of point number, longitude, latitude and altitude, including 7 point measurement records in public sharing mode. The data set contains the field GPS control point joint survey results in Bogda area in 2014, and the file is excel data file. In the data file, longitude, unit degree (°), accurate to 8 decimal places; latitude, unit degree (°), accurate to 8 decimal places; elevation, unit meter (m), accurate to 6 decimal places.

Data source description

Self generated.

Data processing method

This data is the joint survey data of control points from Huace X20 static survey in June 2014, according to the "global positioning system (GPS) survey specification" (GB / t18314-2001), and reference to the "glacier observation and research methods" (Science Press, 2012). There is no national control point, and the synchronous measurement time is more than 1 hour. The results show that the calculation accuracy is controlled within 2cm, and the data longitude and latitude coordinate system UTM WGS84.

Data quality description

4-6 stations of China Survey X20 or T5 GNSS are used for joint survey, the synchronous measurement time is more than 1 hour, and 1-3 national control points are used for joint survey; the result calculation accuracy is controlled within 2cm

Base information

collect time:2014/06/01-2014/06/30

collect place:Bogda

data size:30.3 KiB

data format:

Coordinate system: WGS84

Keywords

Theme:Bogda, GPS, control points
	Subject:Geometric geodesy 
	Time:2014
	Place:Bogda

Space scope

|  | north： 43.87 |  |
| --- | --- | --- |
| west： 88.02 |  | east： 88.47 |
|  | south： 43.69 |  |

Service information

contacts:Li Hongxing

phone:0931-4967592

mailbox:lihongxing@lzb.ac.cn

Contact information

contributors:Wei junfeng,  Bao weijia,  li jing

author:WU Kunpeng

manager:Shangguan donghui , Wei junfeng

Project information

number:2013FY111400，
      name:

Citations and annotations

In order to protect the rights and interests of the platform's scientific and technological resources, expand the services of the platform center, and enhance the application potential of scientific and technological resources,resource users are requested to mark and quote the research results (including published papers, treatises, data products, unpublished research reports, data products, etc.) generated by the use of resources in the following manner.

For the achievements published in English, please refer to the following specifications：

The dataset is provided by National Cryosphere Desert Data Center(https://www.ncdc.ac.cn).

Data reference:

Wei junfeng, Bao weijia, li jing. 2014 Bogda GPS survey data set. National Cryosphere Desert Data Center(http://www.ncdc.ac.cn), 2020. https://cstr.cn/CSTR:11738.11.ncdc.DIMaC.2020.93.