Experimental data set of low temperature drilling fluid treatment agent optimization (January 2019)

DOI：10.12072/ncdc.SRIPE.db1717.2022

CSTR：11738.11.ncdc.SRIPE.db1717.2022

Share type:Login to Access

Category:Others

Datasets description

This dataset was compiled and collected during the implementation of the project: a research dataset on the optimization of low-temperature drilling fluid treatment agents. Indoor experimental methods were used to complete the optimization experiment of low-temperature drilling fluid treatment agents, forming the dataset. Among them, the main elements include apparent viscosity, plastic viscosity, dynamic shear force, filter vector, and density. The indoor experiment of this project started on January 1, 2019 and ended on January 31, 2019, with the collection location in Qingdao, Shandong Province. A total of 36 indoor experiments were conducted, and the experimental dataset was fully open and shared.

Data source description

Independently generated.

Data processing method

Data collection was conducted using a low-temperature test chamber in the laboratory of China University of Petroleum (East China) to create a low-temperature environment. Six speed rotational viscometer, medium pressure filtration apparatus, and drilling fluid were placed in the low-temperature chamber for testing the filtration loss and rheological properties of the drilling fluid. Technical parameters of six speed rotational viscometer: 1. Test range: 1~300mPa • s; 2. Test accuracy: 1-25 mPa • s ± 1mPa. s; 25 mPa • s or more ± 4%; 3. Variable speed range: 3, 6, 100, 200, 300, 600r/min data acquisition API medium pressure filtration loss measuring instrument. Technical parameters of the medium pressure filter: 1. Rated pressure: 0.69MPa; 2. Reasonable total filtration area: 45.6cm θ2; 3. Drilling fluid cup volume: 240mL; Technical parameters of drilling fluid densitometer: 1 Test range: 1-1.5 g/cm θ3; 2. Test accuracy: 0.01 g/cm θ3.

Data quality description

The data quality is good.

Base information

collect time:2019/01/01-2019/01/31

collect place:Qingdao City, Shandong Province

data size:36.5 KiB

data format:Excel

Coordinate system: 无

Keywords

Theme:Low temperature
	Subject:geology 
	Time:2019
	Place:Qingdao City

Space scope

|  | north： |  |
| --- | --- | --- |
| west： |  | east： |
|  | south： |  |

Service information

contacts:Min Yufang

phone:0931-4967596

mailbox:ncdc@lzb.ac.cn

Contact information

contributors:GAO Leli

author:NIU Chengcheng

manager:GAO Leli

Project information

number:2016YFC0303300，
      name:

Citations and annotations

In order to protect the rights and interests of the platform's scientific and technological resources, expand the services of the platform center, and enhance the application potential of scientific and technological resources,resource users are requested to mark and quote the research results (including published papers, treatises, data products, unpublished research reports, data products, etc.) generated by the use of resources in the following manner.

For the achievements published in English, please refer to the following specifications：

The dataset is provided by National Cryosphere Desert Data Center(https://www.ncdc.ac.cn).

Data reference:

GAO Leli. Experimental data set of low temperature drilling fluid treatment agent optimization (January 2019). National Cryosphere Desert Data Center(http://www.ncdc.ac.cn), 2022. https://cstr.cn/CSTR:11738.11.ncdc.SRIPE.db1717.2022.