Heihe River eco hydrological remote sensing experiment: multi-scale observation experiment of surface evapotranspiration on non-uniform underlying surface - flux observation matrix data set (huazhaizi desert station-2012)

DOI：10.12072/ncdc.NIEER.db0083.2020

CSTR：11738.11.ncdc.nieer.2020.1607

Share type:Apply to Access

Category:Remote Sensing

Datasets description

The data set contains the observation data of automatic weather station at point 20 in the flux observation matrix from June 2 to September 21, 2012. The station is huazhaizi station in Zhangye City, Gansu Province, with desert on the underlying surface. The longitude and latitude of the observation point are 100.31860e, 38.76519n and 1731.00m above sea level. Two infrared surface temperature sensors are installed at 2.65m, with the arm facing due south and the probe facing vertically downward; The soil temperature probe is buried at 0cm on the surface and 2cm and 4cm underground, and is 2m away from the south of the meteorological tower; The soil moisture sensor is buried 2cm and 4cm underground, and is 2 meters south of the meteorological tower; Soil heat flow plates (3 pieces) are successively buried 6 cm underground. The data sets of the station are stored in two tables. The time interval of the observation data sets of Beijing Normal University is 10 minutes on average, and the time interval of the observation data sets of the Institute of cold and drought of Chinese Academy of Sciences is 30 minutes on average. The specific observation elements are as follows: （ 1) Observation items: surface radiation temperature (IRT)_ 1、IRT_ 2) (unit: Celsius, percentage), soil heat flux (GS)_ 1、Gs_ 2、Gs_ 3) (unit: watt / m2), multi-layer soil moisture (MS)_ 2cm、Ms_ 4cm (in percentage) and multi-layer soil temperature (TS)_ 0cm、Ts_ 2cm 、Ts_ 4cm) (in degrees Celsius). （ 2) The observation items of cold and drought Institute of huazhaizi Chinese Academy of sciences include: wind speed (WS_ 0.48m、WS_ 0.98m、WS_ 1.99m、WS_ 2.99m (unit: M / s), wind direction (WD)_ 4m (in degrees), dust concentration (funjin)_ mgm3_ AVG), four components of radiation (DR, ur, DLR)_ Cor、ULR_ COR, RN (unit: w / m2), air temperature and humidity observation (TA)_ 1m 、Ta_ 1.99m、Ta_ 2.99m、RH_ 1m、RH_ 1.99m、RH_ 2.99m (unit: Celsius, percentage), pressure (press) (unit: HPA), multi-layer soil temperature (TS)_ 0cm、Ts_ 4cm、Ts_ 10cm、Ts_ 18cm Ts_ 26cm、Ts_ 34cm、Ts_ 42cm、Ts_ 50cm (in degrees Celsius), multi-layer soil moisture percentage (MS)_ 2cm Ms_ 2cm Ms_ 2cm Ms_ 10cm Ms_ 18cm Ms_ 26cm Ms_ 34cm Ms_ 42cm Ms_ 50cm Ms_ 58cm in percent and multilayer soil moisture volume (ADR_ 2cm、ADR_ 2cm、ADR_ 2cm、ADR_ 2cm、ADR_ 10cm、ADR_ 18cm、ADR_ 26cm、ADR_ 34cm、ADR_ 42cm、ADR_ 50cm (unit: m3 / m3), dust momentum (KE)_ Tot (unit: count) and sand particle quantity (PC)_ TOT) (unit: count). Processing and quality control of observation data: 1) ensure that (1) Beijing Normal University observes 1440 data every day (every 10min). If there is a lack of data, it will be marked by - 6999; Ensure that (2) there are 48 data (every 30min) in the cold and drought station of Chinese Academy of Sciences every day. If there is a lack of data, it will be marked by - 6999; 2) Eliminate the time with duplicate records; 3) Data that is obviously beyond the physical meaning or beyond the range of the instrument are deleted; 4) The record time of the observation project of cold and drought Institute of Chinese Academy of Sciences in huazhaizi is world time. The format of date and time is unified, and the date and time are in the same column. For example, the time is: 10:30, June 10, 2012; 5) The naming rule is: AMS + site number. For multi-scale observation test or site information, please refer to Liu et al. (2016), and for observation data processing, please refer to Xu et al. (2013)

Data source description

The station is huazhaizi station in Zhangye City, Gansu Province, with desert on the underlying surface. The longitude and latitude of the observation point are 100.31860e, 38.76519n and 1731.00m above sea level

Data processing method

Two infrared surface temperature sensors are installed at 2.65m, with the arm facing due south and the probe facing vertically downward; The soil temperature probe is buried at 0cm on the surface and 2cm and 4cm underground, and is 2m away from the south of the meteorological tower; The soil moisture sensor is buried 2cm and 4cm underground, and is 2 meters south of the meteorological tower; Soil heat flow plates (3 pieces) are successively buried 6 cm underground. The data sets of the station are stored in two tables. The time interval of the observation data sets of Beijing Normal University is 10 minutes on average, and the time interval of the observation data sets of the Institute of cold and drought of Chinese Academy of Sciences is 30 minutes on average. The specific observation elements are as follows:  （1) Observation items: surface radiation temperature (IRT)_ 1、IRT_ 2) (unit: Celsius, percentage), soil heat flux (GS)_ 1、Gs_ 2、Gs_ 3) (unit: watt / m2), multi-layer soil moisture (MS)_ 2cm、Ms_ 4cm (in percentage) and multi-layer soil temperature (TS)_ 0cm、Ts_ 2cm 、Ts_ 4cm) (in degrees Celsius).  （2) The observation items of cold and drought Institute of huazhaizi Chinese Academy of sciences include: wind speed (WS_ 0.48m、WS_ 0.98m、WS_ 1.99m、WS_ 2.99m (unit: M / s), wind direction (WD)_ 4m (in degrees), dust concentration (funjin)_ mgm3_ AVG), four components of radiation (DR, ur, DLR)_ Cor、ULR_ COR, RN (unit: w / m2), air temperature and humidity observation (TA)_ 1m 、Ta_ 1.99m、Ta_ 2.99m、RH_ 1m、RH_ 1.99m、RH_ 2.99m (unit: Celsius, percentage), pressure (press) (unit: HPA), multi-layer soil temperature (TS)_ 0cm、Ts_ 4cm、Ts_ 10cm、Ts_ 18cm Ts_ 26cm、Ts_ 34cm、Ts_ 42cm、Ts_ 50cm (in degrees Celsius), multi-layer soil moisture percentage (MS)_ 2cm Ms_ 2cm Ms_ 2cm Ms_ 10cm Ms_ 18cm Ms_ 26cm Ms_ 34cm Ms_ 42cm Ms_ 50cm Ms_ 58cm in percent and multilayer soil moisture volume (ADR_ 2cm、ADR_ 2cm、ADR_ 2cm、ADR_ 2cm、ADR_ 10cm、ADR_ 18cm、ADR_ 26cm、ADR_ 34cm、ADR_ 42cm、ADR_ 50cm (unit: m3 / m3), dust momentum (KE)_ Tot (unit: count) and sand particle quantity (PC)_ TOT) (unit: count)

Data quality description

Good data quality

Base information

collect time:2012/06/02-2012/09/21

collect place:Heihe River Basin, artificial oasis experimental area in the middle reaches, flux observation matrix

data size:3.3 MiB

data format:

Coordinate system: WGS84

Keywords

Theme:Hydrometeorology, automatic weather station, meteorological elements, precipitation, atmospheric temperature, precipitation, humidity, air temperature
	Subject:geography 
	Time:2012
	Place:Heihe River Basin,  flux observation matrix,  artificial oasis experimental area in the middle reaches

Space scope

|  | north： 38.77 |  |
| --- | --- | --- |
| west：100.32 |  | east：100.32 |
|  | south： 38.77 |  |

Service information

contacts:Min Yufang

phone:0931-4967596

mailbox:ncdc@lzb.ac.cn

Contact information

contributors:Liu shaomin,  LI Xin,  Xu ziwei

author:Liu shaomin

manager:Xu ziwei

Project information

number:91125002，
      name:

Citations and annotations

In order to protect the rights and interests of the platform's scientific and technological resources, expand the services of the platform center, and enhance the application potential of scientific and technological resources,resource users are requested to mark and quote the research results (including published papers, treatises, data products, unpublished research reports, data products, etc.) generated by the use of resources in the following manner.

For the achievements published in English, please refer to the following specifications：

The dataset is provided by National Cryosphere Desert Data Center(https://www.ncdc.ac.cn).

Data reference:

Liu shaomin, LI Xin, Xu ziwei. Heihe River eco hydrological remote sensing experiment: multi-scale observation experiment of surface evapotranspiration on non-uniform underlying surface - flux observation matrix data set (huazhaizi desert station-2012). National Cryosphere Desert Data Center(http://www.ncdc.ac.cn), 2021. https://cstr.cn/CSTR:11738.11.ncdc.nieer.2020.1607.

Article citation:

Liu Shaomin, Xu Ziwei, Song Lisheng, Zhao Qianyi, Ge Yong, Xu Tongren, Ma Yanfei, Zhu Zhongli, Jia Zhenzhen, Zhang Fen. Upscaling evapotranspiration measurements from multi-site to the satellite pixel scale over heterogeneous land surfaces[J]. Agricultural and Forest Meteorology, 2016, 230: 97-113.

Xu Ziwei, Liu Shaomin, Li Xin, Shi Shengjin, Wang Jiemin, Zhu Zhongli, Xu Tongren, Wang Weizhen, Ma Mingguo. Intercomparison of surface energy flux measurement systems used during the HiWATER‐MUSOEXE[J]. Journal of Geophysical Research, 2013, 118, (23).