Heihe River eco hydrological remote sensing experiment: visible near infrared hyperspectral aerial remote sensing data set (2012)

DOI：10.12072/ncdc.nieer.db3494.2023

CSTR：11738.11.ncdc.NIEER.2021.1766

Share type:Apply to Access

Category:Remote Sensing

Datasets description

This dataset contains (1) Gansu Zhangye CASI + Sasi aerial remote sensing data processing （2）Process of the airborne data_ CASI+SASI On June 29, 2012 and July 7, 2012, in the 30 * 30km core observation area and the midstream sample zone in the middle reaches of Heihe River, the flight test of obtaining visible / near-infrared short wave infrared hyperspectral aerial remote sensing data was carried out by using Yun 12 aircraft and CASI / Sasi sensors. The relative flight altitude is 2000 meters (altitude is 3500 meters). The wavelength ranges of CASI and Sasi sensors are 380-1050 nm and 950-2450 nm respectively, and the spatial resolution is 1 meter and 2.4 meters respectively. The surface reflectance product is obtained by using the synchronously measured ground data and atmospheric data through geometric and 6S atmospheric correction

Data source description

On June 29, 2012 and July 7, 2012, in the 30 * 30km core observation area and the midstream sample zone in the middle reaches of Heihe River, the flight test of obtaining visible / near-infrared short wave infrared hyperspectral aerial remote sensing data was carried out by using Yun 12 aircraft and CASI / Sasi sensors

Data processing method

The relative flight altitude is 2000 meters (altitude is 3500 meters). The wavelength ranges of CASI and Sasi sensors are 380-1050 nm and 950-2450 nm respectively, and the spatial resolution is 1 meter and 2.4 meters respectively. The surface reflectance product is obtained by using the synchronously measured ground data and atmospheric data through geometric and 6S atmospheric correction

Data quality description

Good data quality

Base information

collect time:2012/06/29-2012/06/30

collect place:

data size:302.9 GiB

data format:

Coordinate system: WGS84

Keywords

Theme:CASI, imaging spectrometer, Aerial remote sensing, SASI, Surface reflectance
	Subject:remote sensing geology 
	Time:2012
	Place:Artificial oasis experimental area in the middle reaches,  Heihe River Basin

Space scope

|  | north： 39.00 |  |
| --- | --- | --- |
| west：100.27 |  | east：100.46 |
|  | south： 38.70 |  |

Service information

contacts:

phone:0931-4967596

mailbox:ncdc@lzb.ac.cn

Contact information

contributors:Xiao qing

author:Xiao qing

manager:Xiao qing

Project information

Citations and annotations

In order to protect the rights and interests of the platform's scientific and technological resources, expand the services of the platform center, and enhance the application potential of scientific and technological resources,resource users are requested to mark and quote the research results (including published papers, treatises, data products, unpublished research reports, data products, etc.) generated by the use of resources in the following manner.

For the achievements published in English, please refer to the following specifications：

The dataset is provided by National Cryosphere Desert Data Center(https://www.ncdc.ac.cn).

Data reference:

Xiao qing. Heihe River eco hydrological remote sensing experiment: visible near infrared hyperspectral aerial remote sensing data set (2012). National Cryosphere Desert Data Center(http://www.ncdc.ac.cn), 2021. https://cstr.cn/CSTR:11738.11.ncdc.NIEER.2021.1766.