2012 Dongkemadi glacier subglacial topographic data sets

DOI：10.12072/ncdc.DIMaC.db0071.2020

CSTR：11738.11.ncdc.DIMaC.2020.145

Share type:Apply to Access

Category:Glacier

Datasets description

This data is the terrain data under ice obtained by Mala PROEX ground penetrating radar equipment of Sweden in June 2012; the antenna adopts 100MHz rope antenna; the wave velocity of depth calculation is set to 0.17m/ns; the acquisition software is groundvision2. The data format is excel file format, including serial number, longitude, latitude, thickness, v = 0.17m/ns; the number of data is 5161. Protocol sharing is adopted. The data set includes the 2012 dongkema subglacial topographic survey results in the format of text data file. Features in data file: serial number; longitude (unit: Degree (°)), accurate to 8 decimal places; latitude (unit: Degree (°)), accurate to 8 decimal places; thickness (unit: meter (m)), v = 0.17m/ns, accurate to 2 decimal places.

Data source description

Self generated.

Data processing method

Under-ice terrain data obtained by Mala PROEX ground penetrating radar of Sweden in June 2012, referring to the field survey data of glacier observation and research methods (Science Press, 2012); the antenna adopts 100MHz rope antenna, the wave speed of depth calculation is set to 0.17m/ns, the acquisition software groundvision2, data longitude and latitude coordinate system UTM WGS84.

Data quality description

Before using ground penetrating radar (GPR), the propagation velocity of radar wave velocity in glacier ice is measured, and the parameters of GPR are modified to ensure the most accurate thickness data of corresponding glacier.

Base information

collect time:2012/06/01-2012/06/30

collect place:Dongkemadi

data size:66.9 KiB

data format:

Coordinate system: WGS84

Keywords

Theme:GPR, ice thickness, Dongkemadi
	Subject:Geometric geodesy 
	Time:2012
	Place:Dongkemadi

Space scope

|  | north： 33.33 |  |
| --- | --- | --- |
| west： 91.74 |  | east： 92.27 |
|  | south： 32.78 |  |

Service information

contacts:Li Hongxing

phone:0931-4967592

mailbox:lihongxing@lzb.ac.cn

Contact information

contributors:Shangguan donghui,  Du erji

author:WU Kunpeng

manager:Shangguan donghui

Project information

number:2013FY111400，
      name:

Citations and annotations

In order to protect the rights and interests of the platform's scientific and technological resources, expand the services of the platform center, and enhance the application potential of scientific and technological resources,resource users are requested to mark and quote the research results (including published papers, treatises, data products, unpublished research reports, data products, etc.) generated by the use of resources in the following manner.

For the achievements published in English, please refer to the following specifications：

The dataset is provided by National Cryosphere Desert Data Center(https://www.ncdc.ac.cn).

Data reference:

Shangguan donghui, Du erji. 2012 Dongkemadi glacier subglacial topographic data sets. National Cryosphere Desert Data Center(http://www.ncdc.ac.cn), 2020. https://cstr.cn/CSTR:11738.11.ncdc.DIMaC.2020.145.