2024 Runoff and Sediment Observation Dataset of Yanwachuan River Basin in Loess Plateau Gully Region

DOI：10.12072/ncdc.hwxfzhan.db7018.2025

CSTR：11738.11.NCDC.HWXFZHAN.DB7018.2025

Share type:Apply to Access

Category:Soil and Water Conservation

Datasets description

This dataset is the observation data of runoff and sediment at the Yanwachuan River Basin runoff station from 2021 to 2022. The Yanwachuan River Basin is located in Xifeng District and Ningxian County, Qingyang City, Gansu Province. It is a right bank tributary of the Malian River in the Jing River Basin and a typical loess plateau gully area. As a comprehensive observation station for soil and water conservation benefits in medium-sized watersheds, the experimental observation section of the runoff station is a large parabolic water measuring weir, and the observation method is manual observation. The main observation indicators are daily average flow rate, daily average sediment concentration, daily average sediment transport rate, excerpts of flood hydrological elements, and results of successive flood tests.

Data source description

Manually timed observations

Data processing method

(1) Flow calculation: the observed water level is used to deduce the flow, the formula is Q=20.89H², Q: flow, unit: m³/s; H: water level, unit m.  (2)(2) Calculation of sand content: Calculate the sand content using the formula p=ws/v*1000, p: measured sand content, unit: kg/m³; v: volume of water samples, unit: cm³; ws: weight of dry sand in water samples, unit g.

Data quality description

Observations are compiled and entered through four levels of manual checking, namely, tabulation, initial checking, rechecking, and auditing, to ensure the quality of the data.

Base information

collect time:2024/01/01-2024/12/31

collect place:Yanwachuan River Basin, Ning County, Qingyang City, Gansu Province

altitude:980.0m- 1340.0m

data size:71.1 KiB

data format:

Coordinate system: 无

Keywords

Theme:Average sediment transport rate, erosion modulus, runoff modulus, runoff depth, average flow rate, average sediment concentration
	Subject:
	Time:2024
	Place:Qingyang,  Ningxian,  Yanwachuan,  Gansu Province

Space scope

|  | north： 35.74 |  |
| --- | --- | --- |
| west：107.92 |  | east：107.63 |
|  | south： 35.50 |  |

Service information

contacts:

phone:0931-4967592

mailbox:ncdc@lzb.ac.cn

Contact information

contributors:Yellow River Water Conservancy Commission Xifeng Soil and Water Conservation Scientific Experimental Station

author:YUAN Jing

manager:YUAN Jing

Project information

Citations and annotations

In order to protect the rights and interests of the platform's scientific and technological resources, expand the services of the platform center, and enhance the application potential of scientific and technological resources,resource users are requested to mark and quote the research results (including published papers, treatises, data products, unpublished research reports, data products, etc.) generated by the use of resources in the following manner.

For the achievements published in English, please refer to the following specifications：

The dataset is provided by National Cryosphere Desert Data Center(https://www.ncdc.ac.cn).

Data reference:

Yellow River Water Conservancy Commission Xifeng Soil and Water Conservation Scientific Experimental Station. 2024 Runoff and Sediment Observation Dataset of Yanwachuan River Basin in Loess Plateau Gully Region. National Cryosphere Desert Data Center(http://www.ncdc.ac.cn), 2025. https://cstr.cn/CSTR:11738.11.NCDC.HWXFZHAN.DB7018.2025.