Runoff and sediment observation data set of peijiamaogou watershed in the first sub region of Loess Hilly and gully region from 1960 to 1979

DOI：10.12072/ncdc.HWSDZhan.db0081.2021

CSTR：11738.11.ncdc.HWSDZhan.2021.143

Share type:Login to Access

Category:Soil and Water Conservation

Datasets description

This data set is the runoff and sediment observation data of four runoff stations in peijiamaogou watershed from 1960 to 1979. The runoff and sediment amount during runoff generation are calculated by water level observation and sediment sampling of runoff station. The data set includes the measured large section map of runoff station, the measured discharge result table, the extraction table of flood hydrological elements, the result table of successive flood measurement, the daily average discharge table, the daily average sediment concentration table and the daily average sediment transport rate table. See the basic information table of runoff stations in peijiamaogou basin for the situation of runoff stations.

Data source description

Manual timing observation

Data processing method

1. The water level observation method of runoff station is: read the water gauge number of the flow measurement building on the flow measurement section. In non flood season, 8:00 and 20:00 respectively are observed once a day. If there is rainfall, observe twice at 8:00 and 20:00 respectively when the daily water level changes slowly; observe four times at 2:00, 8:00, 14:00 and 20:00 respectively when the daily water level changes greatly or there are slow peaks and valleys. During the flood season, when the water level changes violently, it shall be observed in multiple of 6 minutes.

Data quality description

Manual observation shall be conducted by professional technicians; during data entry, four levels of manual verification shall be conducted, including tabulation, primary calibration, re calibration and audit, so as to ensure data quality.

Base information

collect time:1960/01/01-1979/12/31

collect place:Peijiamao Valley, Suide County, Yulin City, Shaanxi Province

altitude:2.0m- Nonem

data size:446.7 KiB

data format:Excel

Coordinate system: WGS84

Keywords

Theme:Runoff, Erosion modulus, Runoff depth, Soil loss, Sediment transport rate, water level, flow, Runoff coefficient, Runoff modulus, Sediment concentration
	Subject:Hydrometeorology 
	Time:1960 1961 1962 1963 1964 1965 1966 1967 1968 1969
	Place:Suide,  Peijiamao gully,  Yulin,  Shaanxi Province

Space scope

|  | north： 37.56 |  |
| --- | --- | --- |
| west：110.27 |  | east：110.40 |
|  | south： 37.48 |  |

Service information

contacts:Min Yu Fang

phone:09314967597

mailbox:myf@lzb.ac.cn

Contact information

contributors:Suide administration and Supervision Bureau of water and soil conservation of the Yellow River (Suide scientific experimental station of water and soil conservation)

author:LEI Xin ,  LiuSijun

manager:GUO Yumei , Suide administration and Supervision Bureau of water and soil conservation of the Yellow River (Suide scientific experimental station of water and soil conservation)

Project information

Citations and annotations

In order to protect the rights and interests of the platform's scientific and technological resources, expand the services of the platform center, and enhance the application potential of scientific and technological resources,resource users are requested to mark and quote the research results (including published papers, treatises, data products, unpublished research reports, data products, etc.) generated by the use of resources in the following manner.

For the achievements published in English, please refer to the following specifications：

The dataset is provided by National Cryosphere Desert Data Center(https://www.ncdc.ac.cn).

Data reference:

Suide administration and Supervision Bureau of water and soil conservation of the Yellow River (Suide scientific experimental station of water and soil conservation). Runoff and sediment observation data set of peijiamaogou watershed in the first sub region of Loess Hilly and gully region from 1960 to 1979. National Cryosphere Desert Data Center(http://www.ncdc.ac.cn), 2021. https://cstr.cn/CSTR:11738.11.ncdc.HWSDZhan.2021.143.