Heihe River Integrated Remote Sensing joint test: ENVISAT ASAR ground synchronous observation data set in ARW encrypted observation area during the pre-test period (October 18, 2007)

DOI：10.12072/ncdc.NIEER.db1799.2022

CSTR：11738.11.ncdc.NIEER.2021.1786

Share type:Login to Access

Category:Remote Sensing

Datasets description

On October 18, 2007, the ground synchronous observation test of ENVISAT ASAR data was carried out in ajoufang 1 and ajoufang 2. ENVISAT ASAR data is AP mode, VV / VH polarization combination mode, and the transit time is about 11:17bjt. Both a rou quadrat 1 and a rou quadrat 2 are 3grid × 3grid, 30m for each grid × 30m, a total of 25 sampling points (including center point and corner point). At each sampling point, wet soil moisture meter was used to measure the real part of soil volume water content, electrical conductivity, soil temperature and soil complex dielectric constant; Hand held infrared thermometer to obtain surface radiation temperature; The soil is taken with a 100cm ^ 3 ring knife and dried to obtain the weight water content, soil bulk density and volume water content. At the same time, some parameters of vegetation were investigated, mainly including vegetation height, coverage and vegetation water content. For surface roughness information, please refer to the metadata of "Heihe integrated remote sensing joint test: surface roughness data set of Aru encrypted observation area". This data can provide a basic ground data set for the development and verification of active microwave remote sensing algorithm for retrieving soil moisture and freeze-thaw state.

Data source description

On October 18, 2007, the ground synchronous observation test of ENVISAT ASAR data was carried out in ajoufang 1 and ajoufang 2. ENVISAT ASAR data is AP mode, VV / VH polarization combination mode, and the transit time is about 11:17bjt. Both a rou quadrat 1 and a rou quadrat 2 are 3grid × 3grid, 30m for each grid × 30m, a total of 25 sampling points (including center point and corner point).

Data processing method

At each sampling point, wet soil moisture meter was used to measure the real part of soil volume water content, electrical conductivity, soil temperature and soil complex dielectric constant; Hand held infrared thermometer to obtain surface radiation temperature; The soil is taken with a 100cm ^ 3 ring knife and dried to obtain the weight water content, soil bulk density and volume water content. At the same time, some parameters of vegetation were investigated, mainly including vegetation height, coverage and vegetation water content.

Data quality description

Good data quality.

Base information

collect time:2007/10/18-2007/10/19

collect place:Heihe River Basin, a rou intensive observation area, upstream cold region, hydrological experimental area

data size:60.5 MiB

data format:excel

Coordinate system: WGS84

Keywords

Theme:Aerospace remote sensing, soil, microwave remote sensing, surface radiation temperature, surface radiation characteristics, freezing and thawing, soil moisture, frozen soil, ground verification information, airborne ground remote sensing
	Subject:physical geography 
	Time:2007
	Place:Heihe River Basin,  Aorou encrypted observation area,  Hydrological test area in upper cold region

Space scope

|  | north： 38.97 |  |
| --- | --- | --- |
| west：100.59 |  | east：100.44 |
|  | south： 38.04 |  |

Service information

contacts:Min Yufang

phone:0931-4967596

mailbox:ncdc@lzb.ac.cn

Contact information

contributors:JIN Rui

author:JIN Rui

manager:JIN Rui

Project information

Citations and annotations

In order to protect the rights and interests of the platform's scientific and technological resources, expand the services of the platform center, and enhance the application potential of scientific and technological resources,resource users are requested to mark and quote the research results (including published papers, treatises, data products, unpublished research reports, data products, etc.) generated by the use of resources in the following manner.

For the achievements published in English, please refer to the following specifications：

The dataset is provided by National Cryosphere Desert Data Center(https://www.ncdc.ac.cn).

Data reference:

JIN Rui. Heihe River Integrated Remote Sensing joint test: ENVISAT ASAR ground synchronous observation data set in ARW encrypted observation area during the pre-test period (October 18, 2007). National Cryosphere Desert Data Center(http://www.ncdc.ac.cn), 2022. https://cstr.cn/CSTR:11738.11.ncdc.NIEER.2021.1786.