Observation data set of runoff and sediment in peijiamaogou watershed, the first sub region of Loess Hilly and gully region, 2011 to 2020

DOI：10.12072/ncdc.HWSDZhan.db0055.2020

CSTR：11738.11.ncdc.HWSDZhan.2020.85

Share type:Apply to Access

Category:Soil and Water Conservation

Datasets description

<pre><code> &lt;p&gt;This data set is the observation data of runoff and sediment of 4 runoff stations in peijiamaogou watershed from 2011 to 2020. Through the observation of water level of flow section measured by runoff station and sediment sampling, the runoff and sediment volume at the time of runoff production are calculated. The data set includes the measured large section map of runoff station, the measured discharge result table, the excerpt table of flood hydrological elements, the successive flood test result table, the daily average discharge table, the daily average sediment concentration table and the daily average sediment transport rate table. See the basic situation of runoff station in peijiamaogou watershed for the situation of runoff station. &lt;/p&gt;

Data source description

<pre><code> &lt;p&gt;Manual timing observation&lt;/p&gt;

Data processing method

<pre><code> &lt;p&gt;1. The water level observation method of runoff station is: read the water gauge number of the flow measurement building on the flow measurement section. In non flood season, 8:00 and 20:00 respectively are observed once a day. If there is rainfall, observe twice at 8:00 and 20:00 respectively when the daily water level changes slowly; observe four times at 2:00, 8:00, 14:00 and 20:00 respectively when the daily water level changes greatly or there are slow peaks and valleys. During the flood season, when the water level changes violently, it shall be observed in multiple of 6 minutes.

Data quality description

<pre><code> &lt;p&gt;Manual observation shall be conducted by professional technicians; during data entry, four levels of manual verification shall be conducted, including tabulation, primary calibration, re calibration and audit, so as to ensure data quality. &lt;/p&gt;

Base information

collect time:2011/01/01-2020/12/31

collect place:Peijiamaogou watershed, Suide County, Yulin City, Shaanxi Province

altitude:2.0m- Nonem

data size:3.9 MiB

data format:

Coordinate system: WGS84

Keywords

Theme:Runoff, Erosion modulus, Runoff depth, Soil loss, Sediment transport rate, water level, flow, Runoff coefficient, Runoff modulus, Sediment concentration
	Subject:
	Time:2011 2012 2013 2014 2015 2016 2017 2018 2019 2020
	Place:Suide,  Peijiamao gully,  Yulin,  Shaanxi Province

Space scope

|  | north： 37.56 |  |
| --- | --- | --- |
| west：110.27 |  | east：110.40 |
|  | south： 37.48 |  |

Service information

contacts:Min Yu Fang

phone:093149675976

mailbox:ncdc@lzb.ac.cn

Contact information

contributors:Suide administration and Supervision Bureau of water and soil conservation of the Yellow River (Suide scientific experimental station of water and soil conservation)

author:LEI Xin

manager:GUO Yumei , Suide administration and Supervision Bureau of water and soil conservation of the Yellow River (Suide scientific experimental station of water and soil conservation)

Project information

Citations and annotations

In order to protect the rights and interests of the platform's scientific and technological resources, expand the services of the platform center, and enhance the application potential of scientific and technological resources,resource users are requested to mark and quote the research results (including published papers, treatises, data products, unpublished research reports, data products, etc.) generated by the use of resources in the following manner.

For the achievements published in English, please refer to the following specifications：

The dataset is provided by National Cryosphere Desert Data Center(https://www.ncdc.ac.cn).

Data reference:

Suide administration and Supervision Bureau of water and soil conservation of the Yellow River (Suide scientific experimental station of water and soil conservation). Observation data set of runoff and sediment in peijiamaogou watershed, the first sub region of Loess Hilly and gully region, 2011 to 2020. National Cryosphere Desert Data Center(http://www.ncdc.ac.cn), 2022. https://cstr.cn/CSTR:11738.11.ncdc.HWSDZhan.2020.85.