2023 Runoff and Sediment Observation Dataset of Nanxiaohegou Watershed in Loess Plateau Gully Region

DOI：10.12072/ncdc.hwxfzhan.db6595.2024

CSTR：11738.11.NCDC.HWXFZHAN.DB6595.2024

Share type:Apply to Access

Category:Soil and Water Conservation

Datasets description

& emsp; This dataset is the runoff and sediment observation data of two runoff stations, Huaguoshan Reservoir outlet and Shibamutai, in the Nanxiaohegou watershed of the Loess Plateau gully region in 2023. The Nanxiaohegou watershed is located on the edge of Dongzhiyuan in Xifeng District, Qingyang City, Gansu Province. It is a tributary of the Jinghe River on the left bank of the Puhe River, with a drainage area of 38.93 km2. The main monitoring indicators include the discharge of upstream reservoirs and daily average flow data, daily average sediment concentration data, daily average sediment transport rate data, extracted data of flood hydrological elements, and successive flood test results data of the Nanxiaohegou watershed

Data source description

& emsp; (1) The Eighteen Mu Tai Runoff Station is a control station for the Nanxiaohegou watershed, located at the back slope angle of the Eighteen Mu Tai earth dam, with a catchment area of 32.78km ² above the observation section. In the initial stage of its establishment, the main observation purpose was to observe the changes in runoff and sediment in the watershed after different degrees of treatment. Due to the upstream Huaguoshan Reservoir's interception and storage of some runoff in the watershed, the runoff and sediment data observed by the Eighteen Mu Tai station were only used for comparative analysis of the benefits of reservoir interception and storage. The experimental observation section consists of a Bacher measuring weir and a rectangular thin-walled weir.  & emsp; (2) The outlet flow station of Huaguoshan Reservoir is the control station for the outlet of Huaguoshan Reservoir in the basin, with a catchment area of 26.70km ², mainly observing the reservoir's regulation and storage function and sediment deposition laws. The experimental observation section consists of a Bacher measuring weir and a rectangular thin-walled weir.

Data processing method

& emsp; (1) Flow calculation: Calculate the flow rate based on the observed water level. The calculation formula is Bacher Weir: Q=3.7H1.59, Q: flow rate, unit: m ³/s; H: Water level, unit: m.  & emsp; (2) Calculation of sediment concentration: p=ws/v * 1000 Calculation of sediment concentration, p: measured sediment concentration, unit: kg/m ³; v: Water sample volume, unit: cm ³; WS: Dry sand weight in water sample, unit: g

Data quality description

<p>In the process of data compilation and input, there are four levels of manual verification, including tabulation, primary proofreading, rechecking and auditing, so as to ensure the data quality. </p>

Base information

collect time:2023/01/01-2023/12/31

collect place:Nanxiaohegou Basin, Houguanzhai Township, Xifeng District, Qingyang City, Gansu Province

altitude:1050.0m- 1423.0m

data size:52.9 KiB

data format:

Coordinate system: 无

Keywords

Theme:Average sediment concentration, Average flow, Average sediment transport rate, Runoff depth, Erosion modulus, Runoff modulus
	Subject:
	Time:2023
	Place:Nanxiaohegou watershed,  Qingyang,  Xifeng District,  Gansu Province

Space scope

|  | north： 35.75 |  |
| --- | --- | --- |
| west：107.62 |  | east：107.51 |
|  | south： 35.68 |  |

Service information

contacts:

phone:0931-4967592

mailbox:ncdc@lzb.ac.cn

Contact information

contributors:Yellow River Water Conservancy Commission Xifeng Soil and Water Conservation Scientific Experimental Station

author:YUAN Jing

manager:YUAN Jing

Project information

number:126201340005150007，
      name:

Citations and annotations

In order to protect the rights and interests of the platform's scientific and technological resources, expand the services of the platform center, and enhance the application potential of scientific and technological resources,resource users are requested to mark and quote the research results (including published papers, treatises, data products, unpublished research reports, data products, etc.) generated by the use of resources in the following manner.

For the achievements published in English, please refer to the following specifications：

The dataset is provided by National Cryosphere Desert Data Center(https://www.ncdc.ac.cn).

Data reference:

Yellow River Water Conservancy Commission Xifeng Soil and Water Conservation Scientific Experimental Station. 2023 Runoff and Sediment Observation Dataset of Nanxiaohegou Watershed in Loess Plateau Gully Region. National Cryosphere Desert Data Center(http://www.ncdc.ac.cn), 2024. https://cstr.cn/CSTR:11738.11.NCDC.HWXFZHAN.DB6595.2024.