Runoff data sets of different measures in Yulin Gully Watershed of xindiangou watershed in the first sub region of Loess Hilly and gully region from 1988 to 1990

DOI：10.12072/ncdc.HWSDZhan.db0068.2020

CSTR：11738.11.ncdc.HWSDZhan.2020.128

Share type:Apply to Access

Category:Soil and Water Conservation

Datasets description

This data set is the runoff field data of xindiangou watershed in the first sub region of Loess Hilly and gully region from 1988 to 1990. The precipitation, runoff and sediment yield of runoff generated by different soil and water conservation measures were observed, and the water and sediment reduction benefits of different measures were compared. The runoff and flow field are shown in the table of basic situation of runoff field in Yulin gully.

Data source description

Instrument continuous observation manual observation

Data processing method

1. The observation instruments of secondary rainfall are SM1 ordinary rain gauge with a diameter of 20cm and dsj2 siphon type self recording rain gauge. 2. The runoff and sediment shall be observed manually in the runoff pond, and the water level of the runoff pond shall be read immediately after the end of precipitation and runoff, and the runoff shall be calculated according to the area and water level of the runoff pond; the mud water in the runoff pond shall be stirred evenly to take the sediment sample (500cm3), and the replacement method shall be adopted for sample disposal.

Data quality description

In the process of integration and input of observation data, there are four levels of manual verification, which are tabulation, primary calibration, re calibration and audit, to ensure the data quality.

Base information

collect time:1991/01/01-2000/12/31

collect place:

altitude:840.0m- 1040.0m

data size:66.3 KiB

data format:

Coordinate system: WGS84

Keywords

Theme:Precipitation intensity, Precipitation, runoff, Sediment concentration, Runoff depth, Run off depth, Runoff coefficient
	Subject:
	Time:1991 1992 1993 1994 1995 1996 1997 1998 1999 2000
	Place:Xindiangou,  Yulin,  Suide,  Shaanxi Province

Space scope

|  | north： 37.52 |  |
| --- | --- | --- |
| west：110.28 |  | east：110.30 |
|  | south： 37.49 |  |

Service information

contacts:Min Yu Fang

phone:09314967597

mailbox:myf@lzb.ac.cn

Contact information

contributors:Suide administration and Supervision Bureau of water and soil conservation of the Yellow River (Suide scientific experimental station of water and soil conservation)

author:LEI Xin

manager:GUO Yumei , Suide administration and Supervision Bureau of water and soil conservation of the Yellow River (Suide scientific experimental station of water and soil conservation)

Project information

Citations and annotations

In order to protect the rights and interests of the platform's scientific and technological resources, expand the services of the platform center, and enhance the application potential of scientific and technological resources,resource users are requested to mark and quote the research results (including published papers, treatises, data products, unpublished research reports, data products, etc.) generated by the use of resources in the following manner.

For the achievements published in English, please refer to the following specifications：

The dataset is provided by National Cryosphere Desert Data Center(https://www.ncdc.ac.cn).

Data reference:

Suide administration and Supervision Bureau of water and soil conservation of the Yellow River (Suide scientific experimental station of water and soil conservation). Runoff data sets of different measures in Yulin Gully Watershed of xindiangou watershed in the first sub region of Loess Hilly and gully region from 1988 to 1990. National Cryosphere Desert Data Center(http://www.ncdc.ac.cn), 2020. https://cstr.cn/CSTR:11738.11.ncdc.HWSDZhan.2020.128.