Optical image and X-ray imaging guided imaging guided carbon ion beam therapy technology solutions

DOI：10.12072/ncdc.imp.db3987.2023

CSTR：11738.11.NCDC.IMP.DB3987.2023

Share type:Apply to Access

Category:Others

Datasets description

This report is the design report of the carbon ion therapy technology scheme based on the optical image and X-ray imaging guidance technology of carbon ion therapy technology. The optical image guidance / and X-ray image guidance are jointly debugged and designed to integrate the image data information of optical image and X-ray imaging to improve the accuracy and treatment effect of carbon ion therapy.

Data source description

This scheme relies on optical image and dual plate X-ray imaging system. Firstly, the patient's positioning is verified by comparing the optical image with the body surface information, and then the internal organ position is calibrated by X-ray imaging. Further, the relationship between the body surface information and the internal organ movement is established based on the body surface information and the internal image information, To provide support for the position monitoring and verification of moving target area along with the breathing of patients, the optical image and X-ray image-guided carbon ion beam therapy technology is formed.

Data processing method

The implementation steps of optical image and X-ray image-guided carbon ion beam therapy technology are as follows: (1) CT simulation positioning of simulated treatment room (2) The patient's body surface optical image positioning verification; (3) Double plate X-ray imaging patient positioning verification; (4) Objective to establish the relationship between target movement and respiratory substitute movement; (5) Target motion tracking; (6) Real time monitoring and guidance of patients' body surface information;

Data quality description

Good data quality

Base information

collect time:2017/07/01-2019/12/31

collect place:Lanzhou, Gansu

data size:14.4 MiB

data format:

Coordinate system: 无

Keywords

Theme:Optical image, X-ray guidance, technical feedback
	Subject:
	Time:2017 2018 2019
	Place:Gansu Province,  Gansu

Space scope

|  | north： |  |
| --- | --- | --- |
| west： |  | east： |
|  | south： |  |

Service information

contacts:Min Yufang

phone:0931-4967596

mailbox:ncdc@lzb.ac.cn

Contact information

contributors:XIAO Guoqing

author:XIAO Guoqing

manager:XIAO Guoqing

Project information

number:2017YFC0107500，
      name:Research and implementation of a new conformal intensity modulation technique for multi-particle biology guided by multi-modes

Citations and annotations

In order to protect the rights and interests of the platform's scientific and technological resources, expand the services of the platform center, and enhance the application potential of scientific and technological resources,resource users are requested to mark and quote the research results (including published papers, treatises, data products, unpublished research reports, data products, etc.) generated by the use of resources in the following manner.

For the achievements published in English, please refer to the following specifications：

The dataset is provided by National Cryosphere Desert Data Center(https://www.ncdc.ac.cn).

Data reference:

XIAO Guoqing. Optical image and X-ray imaging guided imaging guided carbon ion beam therapy technology solutions. National Cryosphere Desert Data Center(http://www.ncdc.ac.cn), 2023. https://cstr.cn/CSTR:11738.11.NCDC.IMP.DB3987.2023.