Observation data set of soil moisture content of runoff field in Qiaogou watershed of the first sub region of Loess Hilly and gully region from 2011 to 2020

DOI：10.12072/ncdc.HWSDZhan.db0084.2021

CSTR：11738.11.ncdc.HWSDZhan.db2506.2022

Share type:Apply to Access

Category:Soil and Water Conservation

Datasets description

<pre><code> This data set is the observation data of soil moisture content in runoff field of different landform types in Qiaogou watershed, the first sub region of Loess Hilly and gully region from 2011 to 2020. Qiaogou watershed is located in Suide County, Yulin City, Shaanxi Province, with an area of 0.45 square kilometers. The shape of the watershed is foliar, with a length and width of 1.4km and 0.32km, a relative height difference of 132M, and a gully density of 5.4km2. It belongs to the first tributary of peijiamaogou (the first tributary of Wuding River). In this data set, the soil water content of eight runoff plots with different geomorphic types in Qiaogou was observed. Soil samples were collected every day, and the samples were measured on the day of precipitation, but not on the day of no precipitation. The basic situation of runoff field is shown in the description of runoff field of different landform types in Qiaogou from 2011 to 2020. The main monitoring indicators include: field number, measurement times, sampling date, "soil moisture content (%) sampling depth 0-10cm" and "precipitation between two measurements (mm)".

Data source description

<pre><code> &lt;p&gt;Artificial timing observation&lt;/p&gt;

Data processing method

<pre><code> &lt;p&gt;Precipitation observation instruments include SM1 ordinary rain gauge with a diameter of 20cm and dsj2 siphon type self recording rain gauge. The SM1 type adopts the 8-hour and 20-hour two-stage system for annual observation. It mainly observes the start and end time of precipitation and makes statistics of precipitation. When the intermittent time of precipitation is more than 15 minutes, it is observed as two times of precipitation before and after the interval. When the intermittent time is less than 15 minutes, it is recorded as one-time precipitation observation. If one-time precipitation crosses 8 hours and 20 hours, it should be measured at 8 hours and 20 hours. Dsj2 type adopts 24 section system for flood season observation. In case of failure of self recording rain gauge, the observation data of ordinary rain gauge is used for supplement. &lt;/p&gt;

Data quality description

<pre><code> &lt;p&gt;In order to ensure the data quality, four levels of manual verification are carried out in the process of integrating and entering observation data, which are tabulation, preliminary calibration, re calibration and audit. &lt;P&gt;

Base information

collect time:2011/01/01-2020/12/31

collect place:Qiaogou watershed, Suide County, Yulin City, Shaanxi Province

data size:95.9 KiB

data format:

Coordinate system: WGS84

Keywords

Theme:Soil moisture content, sampling depth, precipitation between two measurements
	Subject:Hydrological survey 
	Time:2011 2012 2013 2014 2015 2016 2017 2018 2019 2020
	Place:Yulin,  Qiaogou,  Suide,  Shaanxi Province

Space scope

|  | north： 37.50 |  |
| --- | --- | --- |
| west：110.29 |  | east：110.30 |
|  | south： 37.49 |  |

Service information

contacts:Min Yu Fang

phone:0931-4967596

mailbox:myf@lzb.ac.cn

Contact information

contributors:Suide administration and Supervision Bureau of water and soil conservation of the Yellow River (Suide scientific experimental station of water and soil conservation)

author:LEI Xin ,  LiuSijun

manager:GUO Yumei , Suide administration and Supervision Bureau of water and soil conservation of the Yellow River (Suide scientific experimental station of water and soil conservation)

Project information

Citations and annotations

In order to protect the rights and interests of the platform's scientific and technological resources, expand the services of the platform center, and enhance the application potential of scientific and technological resources,resource users are requested to mark and quote the research results (including published papers, treatises, data products, unpublished research reports, data products, etc.) generated by the use of resources in the following manner.

For the achievements published in English, please refer to the following specifications：

The dataset is provided by National Cryosphere Desert Data Center(https://www.ncdc.ac.cn).

Data reference:

Suide administration and Supervision Bureau of water and soil conservation of the Yellow River (Suide scientific experimental station of water and soil conservation). Observation data set of soil moisture content of runoff field in Qiaogou watershed of the first sub region of Loess Hilly and gully region from 2011 to 2020. National Cryosphere Desert Data Center(http://www.ncdc.ac.cn), 2021. https://cstr.cn/CSTR:11738.11.ncdc.HWSDZhan.db2506.2022.