Mechanical property data set of granite under real-time high temperature and three-dimensional stress

DOI：10.12072/ncdc.nieer.db2794.2023

CSTR：11738.11.NCDC.NIEER.DB2794.2023

Share type:Apply to Access

Category:Others

Datasets description

<pre><code> &lt;p&gt;&amp;emsp;&amp;emsp; Understanding the mechanical properties of rocks under real-time high temperature and three-dimensional stress is crucial to the thermal extraction of dry hot rocks. This data set mainly studies the influence of temperature and horizontal stress on the strength and deformation behavior of granite. A series of triaxial compression tests were carried out on granite samples at real time and high temperature using a self-developed real time high temperature triaxial test system. The test results show that: (1) With the increase of temperature, the mechanical properties of granite show a trend of first increasing and then decreasing, and their mechanical properties are affected by real-time high temperature, horizontal stress and mineral composition. (2) Under the condition of low horizontal stress, the brittle failure characteristics of granite at room temperature weaken with the increase of stress and temperature. (3) The observation of the fracture morphology of granite samples shows that with the increase of temperature, biotite minerals have toughened, which is an important factor for the reduction of rock brittleness and mechanical strength at high temperatures. (4) Compared with the effect of real-time high temperature pretreatment and high temperature pretreatment on cohesion and friction coefficient, real-time high temperature pretreatment has a greater effect on cohesion, but the friction coefficient has little difference. The test results provide a new idea for understanding the influence of temperature and stress on the mechanical properties of granite.

Data source description

<pre><code> &lt;p&gt;&amp;emsp;&amp;emsp; The data comes from the real-time measurement of the test.

Data processing method

<pre><code> &lt;p&gt;&amp;emsp;&amp;emsp; A series of triaxial compression tests were carried out on granite samples at real time and high temperature using a self-developed real time high temperature triaxial test system.

Data quality description

<pre><code> &lt;p&gt;&amp;emsp;&amp;emsp; The data quality is good.

Base information

collect time:2019/08/01-2019/08/31

collect place:Wuhan, Hubei

data size:304.0 KiB

data format:

Coordinate system: 无

Keywords

Theme:Real time high temperature true triaxial, temperature stress coupling, granite
	Subject:geography 
	Time:2019
	Place:Wuhan

Space scope

|  | north： |  |
| --- | --- | --- |
| west： |  | east： |
|  | south： |  |

Service information

contacts:Minyufang

phone:0931-4967596

mailbox:ncdc@lzb.ac.cn

Contact information

contributors:HU Dawei

author:HU Dawei

manager:HU Dawei

Project information

number:2018YFC0809600，
      name:

Citations and annotations

In order to protect the rights and interests of the platform's scientific and technological resources, expand the services of the platform center, and enhance the application potential of scientific and technological resources,resource users are requested to mark and quote the research results (including published papers, treatises, data products, unpublished research reports, data products, etc.) generated by the use of resources in the following manner.

For the achievements published in English, please refer to the following specifications：

The dataset is provided by National Cryosphere Desert Data Center(https://www.ncdc.ac.cn).

Data reference:

HU Dawei. Mechanical property data set of granite under real-time high temperature and three-dimensional stress. National Cryosphere Desert Data Center(http://www.ncdc.ac.cn), 2023. https://cstr.cn/CSTR:11738.11.NCDC.NIEER.DB2794.2023.