Observation data set of runoff field of different measures in Yulin Gully Watershed of xindiangou watershed in the first sub region of Loess Hilly and gully region from 2011 to 2020

DOI：10.12072/ncdc.HWSDZhan.db0065.2020

CSTR：11738.11.ncdc.HWSDZhan.2020.99

Share type:Apply to Access

Category:Soil and Water Conservation

Datasets description

<p>This data set is the runoff field data of different measures in xindiangou watershed of the first sub region of Loess Hilly and gully region from 2011 to 2018. The secondary precipitation, runoff and sediment yield of runoff generated by different measures of runoff field in Yulin gully were observed, and the benefits of water and sediment reduction by different measures were compared. For the basic situation of runoff field, please refer to the description of runoff field of different measures in Yulin ditch from 2011 to 2020. </p>

Data source description

<p>Manual timing observation</p>

Data processing method

<p>Rainfall observation: 1. There are SM1 ordinary rain gauge with diameter of 20cm and dsj2 siphon self recording rain gauge. Sediment sampling and treatment: different observation facilities are adopted for runoff and sediment, including triangular weir, triangular groove, water distribution tank and runoff pool. Sand ocean is collected by triangular weir and triangular groove in the process of runoff generation; the water level of runoff pool is read immediately after precipitation and runoff is terminated, and runoff is calculated according to the area and water level of water distribution tank and runoff pool The sediment sample (500cm3) was taken by mixing water evenly, and the replacement method was used for sample disposal. Soil moisture content: in a representative fixed site, after removing the surface weeds, samples were taken at a depth of 30 cm and determined by drying (weighing) method.

Data quality description

<p>In the process of compilation and input of observation data, there are four levels of manual verification, including tabulation, primary calibration, recheck and audit, so as to ensure the data quality. </p>

Base information

collect time:2011/01/01-2020/12/31

collect place:Xindiangou watershed, Suide County, Yulin City, Shaanxi Province

altitude:2.0m- Nonem

data size:112.2 KiB

data format:

Coordinate system: WGS84

Keywords

Theme:flow, Soil loss, Runoff, flow, Soil loss, Runoff, Erosion modulus
	Subject:
	Time:2011 2012 2013 2014 2015 2016 2017 2018 2019 2020
	Place:Yulin,  Suide,  Xindiangou,  Shaanxi Province

Space scope

|  | north： 37.52 |  |
| --- | --- | --- |
| west：110.28 |  | east：110.30 |
|  | south： 37.49 |  |

Service information

contacts:Min Yu Fang

phone:09314967597

mailbox:myf@lzb.ac.cn

Contact information

contributors:Suide administration and Supervision Bureau of water and soil conservation of the Yellow River (Suide scientific experimental station of water and soil conservation)

author:LEI Xin

manager:GUO Yumei , Suide administration and Supervision Bureau of water and soil conservation of the Yellow River (Suide scientific experimental station of water and soil conservation)

Project information

Citations and annotations

In order to protect the rights and interests of the platform's scientific and technological resources, expand the services of the platform center, and enhance the application potential of scientific and technological resources,resource users are requested to mark and quote the research results (including published papers, treatises, data products, unpublished research reports, data products, etc.) generated by the use of resources in the following manner.

For the achievements published in English, please refer to the following specifications：

The dataset is provided by National Cryosphere Desert Data Center(https://www.ncdc.ac.cn).

Data reference:

Suide administration and Supervision Bureau of water and soil conservation of the Yellow River (Suide scientific experimental station of water and soil conservation). Observation data set of runoff field of different measures in Yulin Gully Watershed of xindiangou watershed in the first sub region of Loess Hilly and gully region from 2011 to 2020. National Cryosphere Desert Data Center(http://www.ncdc.ac.cn), 2021. https://cstr.cn/CSTR:11738.11.ncdc.HWSDZhan.2020.99.