(corresponding data of the year to be updated) runoff and sediment observation data set of qiaozigou drainage station in the third sub region of Loess Hilly and gully region from 1991 to 2000

DOI：10.12072/ncdc.HWTSZhan.db0038.2020

CSTR：11738.11.ncdc.HWTSZhan.2021.69

Share type:Apply to Access

Category:Soil and Water Conservation

Datasets description

<pre><code> &lt;p&gt;This data set is the runoff and sediment observation data set of qiaozigou watershed in the third sub region of the Loess Hilly and gully region from 1990 to 1999. The runoff and sediment amount during runoff generation are calculated by water level observation and sediment sampling at runoff station. The data set includes the measured discharge results of Qiaozi Donggou and Qiaozi xigougou calibre flow stations, the extract table of flood hydrological elements, the successive flood test results, the daily average discharge table, the daily average sediment concentration table and the daily average suspended sediment transport rate table. &lt;/p&gt;

Data source description

<pre><code> &lt;p&gt;Artificial timing observation&lt;/p&gt;

Data processing method

<pre><code> &lt;ol&gt; Flow observation: the flow measuring structures of qiaozidonggou and qiaozixigou runoff stations are triangular wide crested weirs. When there is little water, the flow is measured by the volume method with the connecting tube, and the flow is calculated by the buoy method for the flood. In flood, straw is used as a buoy, while wood chips and sorghum poles are used as buoys in normal water. The buoy coefficients are not calibrated, and they are all 0.85. The observation time of runoff station is generally twice at 8:00 and 20:00 every day. In flood period, the observation times are increased according to the water regime, generally 5-10 times a day, and 20-40 times for some large floods. Sediment concentration observation: sampling by water side one point method. The number of sampling is basically the same as that of current measurement. Sometimes, the sampling frequency is increased appropriately to control the change of sediment concentration. The filtration method is usually used for sand sample treatment.

Data quality description

<pre><code> &lt;p&gt;Manual observation is carried out by professional and technical personnel; four levels of manual verification are carried out in the process of data input, which are tabulation, initial calibration, re calibration and audit, so as to ensure the data quality. &lt;/p&gt;

Base information

collect time:1991/01/01-2000/12/31

collect place:Qiaozigou watershed, Qinzhou District, Tianshui City, Gansu Province

altitude:1235.0m- 1707.2m

data size:1.6 MiB

data format:

Coordinate system: 无

Keywords

Theme:Annual sediment discharge, annual runoff, total flood, flood sediment discharge, soil loss per unit area, runoff coefficient, daily average discharge, daily average sediment concentration, daily average sediment transport rate, maximum discharge
	Subject:
	Time:1991 1992 1993 1994 1995 1996 1997 1998 1999 2000
	Place:Tianshui,  qiaozigou,  Gansu Province

Space scope

|  | north： 34.63 |  |
| --- | --- | --- |
| west：105.71 |  | east：105.73 |
|  | south： 34.60 |  |

Service information

contacts:

phone:	0931-4967596

mailbox:ncdc@lzb.ac.cn

Contact information

contributors:Yellow River Soil and Water Conservation Tianshui Governance Supervision Bureau (Water and Soil Conservation Tianshui Scientific Experimental Station)

author:Liu Xiao

manager:

Project information

Citations and annotations

In order to protect the rights and interests of the platform's scientific and technological resources, expand the services of the platform center, and enhance the application potential of scientific and technological resources,resource users are requested to mark and quote the research results (including published papers, treatises, data products, unpublished research reports, data products, etc.) generated by the use of resources in the following manner.

For the achievements published in English, please refer to the following specifications：

The dataset is provided by National Cryosphere Desert Data Center(https://www.ncdc.ac.cn).

Data reference:

Yellow River Soil and Water Conservation Tianshui Governance Supervision Bureau (Water and Soil Conservation Tianshui Scientific Experimental Station). (corresponding data of the year to be updated) runoff and sediment observation data set of qiaozigou drainage station in the third sub region of Loess Hilly and gully region from 1991 to 2000. National Cryosphere Desert Data Center(http://www.ncdc.ac.cn), 2022. https://cstr.cn/CSTR:11738.11.ncdc.HWTSZhan.2021.69.