30 meter resolution dataset of impervious surface area and green space ratio in Chinese cities (2000-2018)

DOI：10.5281/zenodo.4034161

CSTR：11738.11.NCDC.ZENODO.DB6528.2024

Share type:Login to Access

Category:Ecology

Datasets description

Urban Impermeable Surface (UIS) and Urban Green Space (UGS) are the two core components that describe the characteristics of the urban underlying environment. However, urban impervious surfaces (UIS) and urban green spaces (UGS) are often embedded together in urban landscapes, with complex structures and composites. Hard classification or binary single type cannot effectively delineate spatially clear urban surface attributes. Although six mainstream datasets of global or national urban land use and land cover products with a spatial resolution of 30 meters have been developed, they only provide binary patterns or dynamics of a single urban land type and cannot effectively divide the quantitative components or structures of urban land cover. Here, we propose a new surveying and mapping strategy that utilizes the advantages of collaborative big data processing and manual interpretation, leveraging geographic knowledge to obtain multi temporal and segmented information on basic types of urban land cover nationwide

Data source description

Vector polygons of urban boundaries in the Chinese Land Use/Cover Dataset (CLUD) derived from satellite imagery for the years 2000, 2005, 2010, 2015, and 2018

Data processing method

Firstly, urban boundary vector polygons for the years 2000, 2005, 2010, 2015, and 2018 were extracted from the China Land Use/Cover Dataset (CLUD) derived from satellite imagery. Secondly, using the Google Earth Engine (GEE) platform, the national settlement and vegetation ratios were retrieved using the Random Forest algorithm and sub-pixel decomposition method. Finally, we developed Chinese UIS and UGS fractional products (CLUD Urban) with 30 meter resolution in 2000, 2005, 2010, 2015, and 2018

Data quality description

We compared our product with six mainstream datasets in terms of quality and accuracy. The evaluation results indicate that compared to other products, the CLUD Urban product has higher accuracy in detecting urban boundaries and urban expansion. In addition, accurate UIS and UGS scores have been developed for each period. The overall accuracy of urban boundaries from 2000 to 2018 exceeded 92.65%; The correlation coefficient (R) and root mean square error (RMSE) of UIS and UGS scores are 0.91 and 0.10 (UIS) and 0.89 and 0.11 (UGS), respectively

Base information

collect time:2000/01/01-2018/12/31

collect place:China

data size:18.2 GiB

data format:tiff

Coordinate system: WGS84

Keywords

Theme:Impermeable surface, land use, random forest algorithm, GEE platform
	Subject:physical geography 
	Time:2000 2001 2002 2003 2004 2005 2006 2007 2008 2009 2010 2011 2012 2013 2014 2015 2016 2017 2018
	Place:China

Space scope

|  | north： 53.50 |  |
| --- | --- | --- |
| west： 73.67 |  | east：135.02 |
|  | south： 3.87 |  |

Service information

contacts:Min Yufang

phone:0931-4967596

mailbox:ncdc@lzb.ac.cn

Contact information

contributors:KUANG Wenhui

author:KUANG Wenhui

manager:KUANG Wenhui

Project information

Citations and annotations

In order to protect the rights and interests of the platform's scientific and technological resources, expand the services of the platform center, and enhance the application potential of scientific and technological resources,resource users are requested to mark and quote the research results (including published papers, treatises, data products, unpublished research reports, data products, etc.) generated by the use of resources in the following manner.

For the achievements published in English, please refer to the following specifications：

The dataset is provided by National Cryosphere Desert Data Center(https://www.ncdc.ac.cn).

Data reference:

KUANG Wenhui. 30 meter resolution dataset of impervious surface area and green space ratio in Chinese cities (2000-2018). National Cryosphere Desert Data Center(http://www.ncdc.ac.cn), 2024. https://cstr.cn/CSTR:11738.11.NCDC.ZENODO.DB6528.2024.

Article citation:

Kuang W, Zhang S, Li X, Lu D. A 30&thinsp;m resolution dataset of China's urban impervious surface area and green space, 2000–2018[J]. Earth Syst. Sci. Data, 2021-01-14, 13, (1): 63-82. 
            DOI: 10.5194/essd-13-63-2021.