Precipitation observation data set of Qiaogou watershed in the first sub region of Loess Hilly and gully region from 2021 to 2022

DOI：10.12072/ncdc.HWSDZhan.db2419.2022

CSTR：11738.11.ncdc.HWSDZhan.db2419.2022

Share type:Apply to Access

Category:Soil and Water Conservation

Datasets description

This data set is the precipitation observation data of Qiaogou watershed rainfall station from 2021 to 2022. Qiaogou Valley is located in Suide County, Yulin City, Shaanxi Province, and belongs to the first level tributary of Peijiamao Valley (the first level tributary of Wuding River). Four rainfall stations, Qiaogou No. 1, Qiaogou No. 2, Qiaogou No. 3 and Qiaogou No. 5, are arranged in the Qiaogou basin. DSJ2 siphon type self recording rain gauge and SM1 ordinary rain gauge with a diameter of 20 cm are used for parallel observation. When the self recording rain gauge fails, ordinary rain gauge is used to replace it. The data includes the layout map of the basin station network, daily precipitation table, precipitation extract table, and maximum precipitation table in each period. See the Description of Qiaogou Watershed from 2021 to 2022 for the basic information of rainfall stations in Qiaogou Watershed

Data source description

Continuous observation by instrument and regular observation manually

Data processing method

Precipitation observation instruments include DSJ2 siphon self recording rain gauge and SM1 ordinary rain gauge with a diameter of 20cm. The DSJ2 self recording rain gauge adopts the 24 segment system for flood season observation, and the SM1 ordinary rain gauge adopts the 8:00 and 20:00 two segment system for annual observation. In case of failure of the self recording rain gauge, the observation data of the ordinary rain gauge shall be used for supplement. The starting and ending time of precipitation is mainly observed, and the precipitation is counted. When the interval time of precipitation is more than 15min, it is observed as two times of precipitation before and after the interval time. When the interval time is less than 15min, it is recorded as a precipitation observation. If a precipitation exceeds 8 and 20 hours, it should be added at 8 and 20 hours

Data quality description

The recording rain gauge shall be calibrated before the start of observation every year. In the process of data entry, there are four levels of manual verification: tabulation, initial verification, re verification, and audit to ensure data quality

Base information

collect time:2021/01/01-2022/12/31

collect place:Qiaogou watershed, Suide County, Yulin City, Shaanxi Province

data size:196.0 KiB

data format:

Coordinate system: WGS84

Keywords

Theme:Daily precipitation, precipitation intensity, maximum precipitation, period precipitation
	Subject:geography , hydrology 
	Time:2021 2022
	Place:Qiaogou,  Yulin,  Suide,  Shaanxi Province

Space scope

|  | north： 37.50 |  |
| --- | --- | --- |
| west：110.29 |  | east：110.30 |
|  | south： 37.49 |  |

Service information

contacts:Min Yu Fang

phone:09314967596

mailbox:myf@lzb.ac.cn

Contact information

contributors:Suide administration and Supervision Bureau of water and soil conservation of the Yellow River (Suide scientific experimental station of water and soil conservation)

author: ,

manager:Suide administration and Supervision Bureau of water and soil conservation of the Yellow River (Suide scientific experimental station of water and soil conservation) , LEI Xin

Project information

number:126201340005150007，
      name:

Citations and annotations

In order to protect the rights and interests of the platform's scientific and technological resources, expand the services of the platform center, and enhance the application potential of scientific and technological resources,resource users are requested to mark and quote the research results (including published papers, treatises, data products, unpublished research reports, data products, etc.) generated by the use of resources in the following manner.

For the achievements published in English, please refer to the following specifications：

The dataset is provided by National Cryosphere Desert Data Center(https://www.ncdc.ac.cn).

Data reference:

Suide administration and Supervision Bureau of water and soil conservation of the Yellow River (Suide scientific experimental station of water and soil conservation). Precipitation observation data set of Qiaogou watershed in the first sub region of Loess Hilly and gully region from 2021 to 2022. National Cryosphere Desert Data Center(http://www.ncdc.ac.cn), 2022. https://cstr.cn/CSTR:11738.11.ncdc.HWSDZhan.db2419.2022.