2008 joint survey data set of GPS control points in Tuomuer peak area of Tianshan Mountains

DOI：10.12072/ncdc.DIMaC.db0075.2020

CSTR：11738.11.ncdc.DIMaC.2020.77

Share type:Apply to Access

Category:Glacier

Datasets description

This data set is for the area of Mount Tomur in Tianshan, mainly including the glacier in keqihar in Tianshan, and the joint measurement data set of GPS control points includes the serial number, longitude, latitude and elevation of the control points. The data set contains 374 records, which are mainly obtained by purchasing the national primary control points in the region, using the national primary control points for static measurement, synchronous measurement time of more than 1 hour, obtaining the glacier area control points through solution, and dynamic joint measurement through DGPS RTK. The data set is fully open sharing.

Data source description

This data set is measured data, and the main data sources are static measurement of national primary control points and dynamic joint measurement of DGPS RTK.

Data processing method

Through purchasing the national primary control points in the region, the global positioning system (GPS) of Huace T5 model is used for synchronous measurement for more than 1 hour, and the control points in the glacier area are obtained through the solution of hCG software; then the dynamic differential GPS is used to obtain the dynamic joint measurement, and the dynamic differential GPS measurement process is standardized and standardized, and strictly reviewed.

Data quality description

4-6 stations of CHCNAV X20 or T5 GNSS are used for joint survey, the synchronous measurement time is more than 1 hour, and 1-3 national control points are used for joint survey; the result calculation accuracy is controlled within 2cm

Base information

collect time:2008/08/01-2008/08/31

collect place:Tuomuer peak area, Tianshan Mountains

data size:40.1 KiB

data format:

Coordinate system: WGS84

Keywords

Theme:GPS, control point, Mount Tianshan
	Subject:
	Time:2008
	Place:The keqikar glacier of tuomu'er peak in Tianshan Mountains

Space scope

|  | north： 41.68 |  |
| --- | --- | --- |
| west： 80.08 |  | east： 80.19 |
|  | south： 41.78 |  |

Service information

contacts:Li Hongxing

phone:0931-4967592

mailbox:lihongxing@lzb.ac.cn

Contact information

contributors:LIU Shiyin,  Shangguan donghui,  Jiang zongli,  Zhang yisong

author:WU Kunpeng

manager:WU Kunpeng , LIU Shiyin

Project information

number:2013FY111400，
      name:

Citations and annotations

In order to protect the rights and interests of the platform's scientific and technological resources, expand the services of the platform center, and enhance the application potential of scientific and technological resources,resource users are requested to mark and quote the research results (including published papers, treatises, data products, unpublished research reports, data products, etc.) generated by the use of resources in the following manner.

For the achievements published in English, please refer to the following specifications：

The dataset is provided by National Cryosphere Desert Data Center(https://www.ncdc.ac.cn).

Data reference:

LIU Shiyin, Shangguan donghui, Jiang zongli, Zhang yisong. 2008 joint survey data set of GPS control points in Tuomuer peak area of Tianshan Mountains. National Cryosphere Desert Data Center(http://www.ncdc.ac.cn), 2020. https://cstr.cn/CSTR:11738.11.ncdc.DIMaC.2020.77.