Runoff and sediment observation data set of Leek Garden Gully watershed in the first sub-region of Loess Hills and Gullies area from 1991 to 2000

DOI：10.12072/ncdc.HWSDZhan.db0043.2020

CSTR：11738.11.ncdc.HWSDZhan.2020.117

Share type:Apply to Access

Category:Soil and Water Conservation

Datasets description

Data source description

Artificial timing observation

Data processing method

1. Water level observation: Jiuyuangou runoff station uses the water gauge on the side wall of the building to measure the water level. In non flood season, the water level is observed once at 8:00 a.m. in case of rainfall, the daily water level changes slowly twice at 8:00 and 20:00; when the daily water level changes greatly or there is a slow peak and valley, it is observed four times at 2:00, 8:00, 14:00 and 20:00. When the water level changes violently in flood season, the observation shall be made in multiples of 6 minutes. 2. Flow measurement method: when the water level is higher than the delta channel, the flow measurement method is adopted in the rectangular spillway, and the buoy coefficient is the buoy test results of huangweizizhou runoff test station. 3. Sediment sampling and treatment: the midstream buoy method is used to sample the sediment at the flow measurement section. The frequency of sediment sampling depends on the frequency of flood water level change, and the sampling frequency is increased to control the change of sediment concentration at flood peak. The sample was treated by displacement method.

Data quality description

Manual observation is carried out by professional and technical personnel; four levels of manual verification are carried out in the process of data input, which are tabulation, initial calibration, re calibration and audit, so as to ensure the data quality.

Base information

collect time:1991/01/01-2000/12/31

collect place:Jiuyuangou watershed, Suide County, Yulin City, Shaanxi Province

altitude:2.0m- Nonem

data size:331.1 KiB

data format:xls，doc，pdf

Coordinate system: WGS84

Keywords

Theme:Runoff modulus, runoff coefficient, sediment transport rate, water level, erosion modulus, discharge, sediment concentration, runoff depth, runoff, soil loss
	Subject:
	Time:1991 1992 1993 1994 1995 1996 1997 1998 1999 2000
	Place:Jiuyuangou,  Suide,  Yulin,  Shaanxi Province

Space scope

|  | north： 37.62 |  |
| --- | --- | --- |
| west：110.26 |  | east：110.39 |
|  | south： 37.54 |  |

Service information

contacts:Min Yu Fang

phone:093149675976

mailbox:myf@lzb.ac.cn

Contact information

contributors:Suide administration and Supervision Bureau of water and soil conservation of the Yellow River (Suide scientific experimental station of water and soil conservation)

author:LEI Xin ,  LiuSijun

manager:GUO Yumei , Suide administration and Supervision Bureau of water and soil conservation of the Yellow River (Suide scientific experimental station of water and soil conservation)

Project information

Citations and annotations

In order to protect the rights and interests of the platform's scientific and technological resources, expand the services of the platform center, and enhance the application potential of scientific and technological resources,resource users are requested to mark and quote the research results (including published papers, treatises, data products, unpublished research reports, data products, etc.) generated by the use of resources in the following manner.

For the achievements published in English, please refer to the following specifications：

The dataset is provided by National Cryosphere Desert Data Center(https://www.ncdc.ac.cn).

Data reference:

Suide administration and Supervision Bureau of water and soil conservation of the Yellow River (Suide scientific experimental station of water and soil conservation). Runoff and sediment observation data set of Leek Garden Gully watershed in the first sub-region of Loess Hills and Gullies area from 1991 to 2000. National Cryosphere Desert Data Center(http://www.ncdc.ac.cn), 2020. https://cstr.cn/CSTR:11738.11.ncdc.HWSDZhan.2020.117.